| Declaração de Conformidade EASA-OPS Part CAT.IDE Statement of Compliance with EASA-OPS Part CAT.IDE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operador Operator |  | Registo Registration |  | Modelo Aeronave Aircraft Model |  | Número série Serial Number | Data fabrico Manufacture date |
| MCTOM Maximum Certified Take-off Mass |  | MOPSC Maximum Operational Passenger Seating Configuration |  | Data do 1º CofA Date of 1st Cofa |  | TCDS Aeronave Aircraft TCDS | TCDS Motor Engine TCDS |
| Tipo de Operações Type of operations |  |  |  |  |  |  |  |
| IFR ☐ VFR ☐ | Dia/Noite ☐ Day/night Dia ☐ Day |  | Cond. de Gelo ☐ Icing conditions Sem cond. de gelo ☐ Non Icing Cond. |  | Sobre água ☐ Over water Sobre terra ☐ Over land |  | Duplo comando ☐ Dual Command Comando único ☐ Single Pilot |
| Aeronave Aircraft |  |  |  |  |  |  |  |
| Pressurizada ☐ Pressurized Não pressurizada ☐ Non Pressurized |  | Altitude máxima Max. Altitude |  | OAT intervalo OAT interval |  |  |  |
| Outra informação relevante: Other relevant information: |  |  |  |  |  |  |  |
| Documentação que sustenta o pedido Documents that support the request |  |  |  |  |  |  |  |
| Refª Publicação Publication Reference | Descrição ou nome da publicação Description or name of the publication |  |  |  |  |  |  |

# TABELA 1

| Anexo Annex | Relatórios Reports |
| --- | --- |
|  | CVR conforme AMC |
|  | Gravação de dados do FDR conforme CAT.IDE.A.190 Flight data recorder |
|  | Gravação de dados do Data Link conforme CAT.IDE.A.195 Data link recording |
|  | Gravação de dados do Combination Recorder CAT.IDE.A.200 Combination recorder |

Regras de preenchimento

Seguidamente descreve-se como é composto o documento e qual deverá ser a forma de preenchimento do mesmo.

O requerente (operador/proprietário) deve preencher o campo 3 – “Método de cumprimento ou razão para não cumprimento – Operador”, com base nos GM e AMC em vigor. Nos campos   ☐ N/A    ☐ Compliant   ☐ Non Compliant deverá também identificar se cumpre, não é aplicável ou não cumpre o requisito.

No campo 3, o requerente deve indicar, de forma clara:

- o método de cumprimento adotado para cada requisito, ou

- a razão para o não cumprimento, quando aplicável.

Sempre que a justificação do operador/proprietário seja suportada por documentação específica, deverá:

- identificar o documento no campo 3 (por exemplo, título, referência e data), e

- anexar esse documento ao requerimento, de forma a fundamentar o cumprimento do requisito.

Deverá ainda preencher a Declaração de Conformidade, datando e assinando a mesma.

Apenas necessita enviar o documento referente ao Anexo para o qual está a submeter a aplicação, isto é, se for para Helicóptero-Anexo II não necessita submeter o Anexo I referente a Aviões, e vice-versa.

| Requisito1 Requirement | Requisito Específico2 Specific Requirement | Método de cumprimento ou razão para não cumprimento-Operador3 Method of compliance or Reason for Non applicability-Operator |
| --- | --- | --- |
| CAT.IDE.A.100 - Instruments and equipment – general ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Single instrument (required to be installed in an aeroplane operated in a multi-crew environment), to be readily operable and visible from each flight crew station. - All required emergency equipment to be easily accessible for immediate use. |  |  |
| CAT.IDE.A.105 - Minimum equipment for flight ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| A flight shall not be commenced when any of the aeroplane’s instruments, items of equipment or functions required for the intended flight are inoperative or missing, unless either of the followingconditions is fulfilled: - the aeroplane is operated in accordance with the minimum equipment list (MEL); - for complex motor-powered aeroplanes and for any aeroplane used in commercial operations, the operator is approved by the competent authority to operate the aeroplane within the constraints of the master minimum equipment list (MMEL) in accordance with point ORO.MLR.105(j) of Annex III; - the aeroplane is subject to a permit to fly issued in accordance with the applicable airworthiness requirements. |  |  |
| ORO.GEN.110(d) ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Equipment required for the approved area of operations: The operator shall ensure that its aircraft are equipped and its crew are qualified as required for the area and type of operation. | - Check that specific equipment requirement related to the area of operations and/or operated routes are met (check ICAO Doc 7030)/ |  |
| CAT.IDE.A.110 - Spare electrical fuses ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Number to be the highest of : - 10% of the number of fuses of each rating - 3 fuses for each rating |  |  |
| CAT.IDE.A.115 - Operating lights ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Operating by day: - an anti-collision light system; - lighting supplied from the aeroplane’s electrical system to provide adequate illumination for all instruments and equipment essential to the safe operation of the aeroplane; - lighting supplied from the aeroplane’s electrical system to provide illumination in all passenger compartments; and - an independent portable light for each required crew member readily accessible to crew members when seated at their designated stations. Aeroplanes operated at night shall in addition be equipped with: - Navigation/position light; - 2 landings lights or one single light having 2 separately energised filament; - Seaplane light (to conform with international regulations for preventing collisions at sea). |  |  |
| CAT.IDE.A.120 - Equipment to clear windshield ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Means to maintain a clear portion of the windshield during precipitation (windshield wipers or equivalent). |  |  |
| CAT.IDE.A.125 - Operations under VFR by day – flight and navigational instruments and associated equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - A means of measuring and displaying: - Magnetic heading; - Time in hours, minutes and seconds; - Indicated airspeed (in kts); - Vertical speed; - Turn and slip, or in the case of aeroplanes equipped with a standby means of measuring and displaying attitude, slip; - Attitude; - Stabilised heading; - Outside air temperature (in °C); and - Mach number whenever speed limitations are expressed in terms of Mach number. - When 2 pilots are required for the operation, a separate means of displaying for the 2nd pilot: - Barometric altitude; - Indicated airspeed; - Vertical speed; - Turn and slip; - Attitude; and - Stabilised heading. -A means of indicating when the supply of power to the required flight instruments is not adequate. -A means for preventing malfunction of the required airspeed indicating systems due to condensation or icing (heated pitot tube or equivalent). - Two means of measuring and displaying barometric altitude (in ft with sub-scale setting in mb/hpa) - A means of annunciating to the FC the failure of the above means (pitot heating) for aeroplanes: - issued individual CofA on or after 01/04/98; or - issued individual CofA before 01/04/98 with an MCTOM > 5 700 kg, and with an MOPSC > 9. - Except for propeller-driven aeroplanes with an MCTOM ≤ 5 700 kg, two independent static pressure systems. - One static pressure system and one alternate source of static pressure for propeller-driven aeroplanes with an MCTOM ≤ 5 700 kg. - A standby (illuminated) means of measuring and displaying attitude capable of being used from either pilot’s station for aeroplanes with an MCTOM > 5 700 kg or an MOPSC > 9 that: - is powered continuously during normal operation and, after a total failure of the normal electrical generating system, is powered from a source independent from the normal electrical generating system; - provides reliable operation for a minimum of 30 mn after total failure of the normal electrical generating system, taking into account other loads on the emergency power supply and operational procedures; - operates independently of any other means of measuring and displaying attitude; - is operative automatically after total failure of the normal electrical generating system; - is appropriately illuminated during all phases of operation, except for aeroplanes with an MCTOM ≤ 5 700 kg, already registered in a MS on 01/04/95 and equipped with a standby attitude indicator in the left-hand instrument panel; - is clearly evident to the FC when the standby attitude indicator is being operated by emergency power; and - where the standby attitude indicator has its own dedicated power supply, has an associated indication, either on the instrument or on the instrument panel, when this supply is in use. - A chart holder in an easily readable position that can be illuminated for night operations. |  |  |
| CAT.IDE.A.130 - Operations under IFR or at night – flight and navigational instruments and associated equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -A means of measuring and displaying: - Magnetic heading; - Time in hours, minutes and seconds; - Indicated airspeed; - Vertical speed; - Turn and slip, or displaying attitude, slip; - Attitude; - Stabilised heading; - Outside air temperature; - Mach number whenever speed limitations are expressed in terms of Mach number. -Two means of measuring and displaying barometric altitude. -A means of indicating when the supply of power to the required flight instruments is not adequate. -A means for preventing malfunction of the airspeed indicating systems required in (a)(3) and (h)(2) due to condensation or icing. -A means of annunciating to the flight crew the failure of the means required in (d) for aeroplanes: - issued with an individual CofA on or after 1 April 1998; or - issued with an individual CofA before 1 April 1998 with an MCTOM of more than 5 700 kg, and with an MOPSC of more than nine. -Except for propeller-driven aeroplanes with an MCTOM of 5 700 kg or less, two independent static pressure systems. -One static pressure system and one alternate source of static pressure for propeller-driven aeroplanes with an MCTOM of 5 700 kg or less. -Whenever 2 pilots are required for the operation, a separate means of displaying for the 2nd pilot: - Barometric altitude; - Indicated airspeed; - Vertical speed; - Turn and slip; - Attitude; - Stabilised heading. -A standby means of measuring and displaying attitude capable of being used from either pilot’s station for aeroplanes with an MCTOM of more than 5 700 kg or an MOPSC of more than nine that: - is powered continuously during normal operation and, after a total failure of the normal electrical generating system, is powered from a source independent from the normal electrical generating system; - provides reliable operation for a minimum of 30 minutes after total failure of the normal electrical generating system, taking into account other loads on the emergency power supply and operational procedures; - operates independently of any other means of measuring and displaying attitude; - is operative automatically after total failure of the normal electrical generating system; - is appropriately illuminated during all phases of operation, except for aeroplanes with an MCTOM of 5 700 kg or less, already registered in a Member State on 1 April 1995 and equipped with a standby attitude indicator in the left-hand instrument panel; - is clearly evident to the flight crew when the standby attitude indicator is being operated by emergency power; and - where the standby attitude indicator has its own dedicated power supply, has an associated indication, either on the instrument or on the instrument panel, when this supply is in use. -A chart holder in an easily readable position that can be illuminated for night operations. |  |  |
| CAT.IDE.A.135- Additional equipment for single-pilot operation under IFR ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Autopilot with at least altitude hold and heading mode. |  |  |
| CAT.IDE.A.140- Altitude alerting system ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Altitude alerting system capable of: - Alerting FC when approaching a selected altitude; - Alerting FC (aural signal) when deviating from a preselected altitude. | - Applicable only to: Turbine propeller aeroplanes with an MCTOM > 5 700 kg or having an MOPSC > 9; And Aeroplanes powered by turbo-jet engines. |  |
| CAT.IDE.A.150- Terrain awareness warning system (TAWS) ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Class A TAWS for turbine-powered aeroplanes having an MCTOM > 5 700 kg or an MOPSC > 9 - Class B TAWS for reciprocating-engine-powered aeroplanes with an MCTOM > 5 700 kg or an MOPSC > 9 - Class B TAWS for turbine-powered aeroplanes for which the individual CofA was first issued after 01/01/19 and having an MCTOM ≤ 5 700 kg and an MOPSC of 6 to 9 |  |  |
| CAT.IDE.A.155 - Airborne collision avoidance system (ACAS) ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Applicable to all turbine-powered aeroplanes with an MCTOM > 5 700 kg or an MOPSC > 19 | - Check that version 7.1 is installed. |  |
| CAT.IDE.A.160 Airborne weather detecting equipment: ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Applicable to: - Pressurised aeroplanes; - Non-pressurised aeroplanes with an MCTOM > 5 700 kg; and - Non-pressurised aeroplanes with an MOPSC > 9. | - for propeller driven pressurised aeroplanes with an MCTOM ≤5 700 kg and an MOPSC ≤ 9, equipment capable of detecting thunderstorms and other potentially hazardous weather conditions, regarded as detectable with airborne weather radar equipment, are also acceptable. |  |
| CAT.IDE.A.165 - Additional equipment for operations in icing conditions at night ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - means to illuminate or detect the formation of ice, which should not cause glare or reflection that would handicap crew members in the performance of their duties |  |  |
| CAT.IDE.A.170 - Flight crew interphone system ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Flight crew interphone system, including headsets and microphones for use by all FC members for aeroplanes operated by more than one FC member. |  |  |
| CAT.IDE.A.175 - Crew member interphone system ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Independent from PA (except for handset, headsets, microphones, switches and signalling devices); - Fitted with an alerting system (aural or visual) (if applicable) - Differentiation between normal and emergency calls. - 2 way communication between: FC compartment and: - FC compartment and each passenger compartment - FC compartment and each galley (other than on passenger deck) - FC compartment and each remote crew compartment not on passenger deck; - At least 2 FC members and ground personnel. - Accessible from CC stations (if applicable); - Fitted with an alerting system (aural or visual) (if applicable) - Differentiation between normal and emergency calls. | - Applicable to Aeroplanes with an MCTOM > 15 000 kg, or with an MOPSC > 19, except for aeroplanes first issued with an individual CofA before 01/04/65 and already registered in a MS on 01/04/95. |  |
| CAT.IDE.A.180- Public address system ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - For aeroplanes with an MOPSC > 19. - Independent from interphone system (except for handset, headsets, microphones, switches and signalling devices). - A microphone for each floor level passenger emergency exit with an adjacent CC seat. - Operable within 10 s by a CC at those stations. - Audible at all passenger seats, lavatories, galleys, CC seats, work stations and remote crew areas. |  |  |
| CAT.IDE.A.185 - Cockpit voice recorder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Applicable to: - aeroplanes with an MCTOM > 5 700 kg; and - Multi-engined turbine-powered aeroplanes with an MCTOM ≤ 5 700 kg with an MOPSC > 9 and first issued with an individual CofA on or after 01/01/90. Recording duration: - the preceding 25 h for aeroplanes with an MCTOM > 27 000 kg and first issued with an individual CofA on or after 01/01/22; or - the preceding 2 hours in all other cases. - Other means than magnetic tape or magnetic wire. - Capable of recording: - FC voice communications in cockpit through the radio; - FC voice communications in cockpit through the interphone system and PA; - Aural environment of the FC compartment: - Voice and audio signals identifying nav or approach aids in headset or speaker. - If not deployable, with an ULD with a minimum transmitting time of 90 days. - If deployable, with an ELT. - Aeroplane with an MCTOM over 27.000 kg and first issued with an individual CoA on or after 05/09/22 to be equipped with an alternate power source for the CVR and cockpit mounted area microphone. |  |  |
| CAT.IDE.A.190 - Flight data recorder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Applicable to: - Aeroplanes with an MCTOM > 5 700 kg and first issued with an individual CofA on or after 01/06/90; - Turbine-engined aeroplanes with an MCTOM > 5 700 kg and first issued with an individual CofA before 01/06/90; and - Multi-engined turbine-powered aeroplanes with an MCTOM ≤ 5 700 kg, with an MOPSC > 9 and first issued with an individual CofA on or after 01/04/98. - Recording capacity: - time, altitude, airspeed, normal acceleration and heading; data recorded to be retained during at least the preceding 25 hours for aeroplanes referred to bullet point 2 above with an MCTOM of less than 27 000 kg; - the parameters required to determine accurately the aeroplane flight path, speed, attitude, engine power and configuration of lift and drag devices; data recorded to be retained during at least the preceding 25 hours, for aeroplanes referred to in bullet point 1 above with an MCTOM of less than 27 000 kg and first issued with an individual CofA before 01/01/2016; - the parameters required to determine accurately the aeroplane flight path, speed, attitude, engine power and configuration of lift and drag devices; data recorded to be retained during at least the preceding 10 hours, in the case of aeroplanes referred to in bullet point 3 above and first issued with an individual CofA before 01/01/2016; or - the parameters required to determine accurately the aeroplane flight path, speed, attitude, engine power, configuration and operation; data recorded to be retained during at least the preceding 25 hours, for aeroplanes referred to in bulet points 1 and 3 above and first issued with an individual CofA on or after 01/01/2016. - Data to be obtained from aeroplane sources that enable accurate correlation with information displayed to the FC. - FDR to start to record the data prior to the aeroplane being capable of moving under its own power and to stop after the aeroplane is incapable of moving under its own power. In addition, in the case of aeroplanes issued with an individual CofA on or after 01/04/1998, FDR to start automatically to record the data prior to the aeroplane being capable of moving under its own power and to stop automatically after the aeroplane is incapable of moving under its own power. - If the FDR is not deployable, it shall have a device to assist in locating it under water. By 16/06/2018 at the latest, this device shall have a minimum underwater transmission time of 90 days. If the FDR is deployable, it shall have an automatic emergency locator transmitter. | - Check actual list of parameters recorded against AMCs to CAT.IDE.A.190 |  |
| CAT.IDE.A.191 - Lightweight flight recorder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Lightweight flight recorder: - Applicable to turbine-engined aeroplanes with an MCTOM ≥ 2 250 kg and aeroplanes with an MOPSC > 9 if all of the following conditions are met: - not within the scope of point CAT.IDE.A.190(a); - first issued with an individual CofA on or after 05/09/2022. - Information recorded sufficient to determine the flight path and aircraft speed. - Capable of recording flight data or images recorded during at least the last 5 hours. - Flight recorder to automatically start to record prior to the aeroplane being capable of moving under its own power and to stop automatically after the aeroplane is no longer capable of moving under its own power. - If the flight recorder records images or audio of the flight crew compartment, then a function shall be provided which can be operated by the commander and which modifies image and audio recordings made before the operation of that function, so that those recordings cannot be retrieved using normal replay or copying techniques. | - If the flight recorder records flight data, check the list of parameters to be recorded in AMC1 CAT.IDE.A.191. - If images are recorded, check the criteria applicable to such images in AMC1 CAT.IDE.A.191. |  |
| CAT.IDE.A.195 - Data link recording ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Applicable to aeroplanes first issued with an individual CofA on or after 08/04/14 that have the capability to operate data link communications and are required to be equipped with a CVR. - Information to be recorded: - Data link communication messages related to ATS communications to and from the aeroplane, - Information that enables correlation to any associated records related to data link communications and stored separately from the aeroplane; and - Information on the time and priority of data link communications messages, taking into account the system’s architecture. |  |  |
| CAT.IDE.A.200 - Combination recorder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Compliance with CVR and FDR requirements may be achieved by: - one flight data and cockpit voice combination recorder in the case of aeroplanes required to be equipped with a CVR or an FDR; - one flight data and cockpit voice combination recorder in the case of aeroplanes with an MCTOM of 5 700 kg or less and required to be equipped with a CVR and an FDR; - or two flight data and cockpit voice combination recorders in the case of aeroplanes with an MCTOM of more than 5 700 kg and required to be equipped with a CVR and an FDR. |  |  |
| CAT.IDE.A.205 - Seats, seat safety belts, restraint systems and child restraint devices ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Flight crew seats: - a seat belt with UTR system incorporating a device that will automatically restrain the occupant’s torso in the event of rapid deceleration: (i) on each FC seat and on any seat alongside a pilot’s seat; (ii) on each observer seat located in the FC compartment; - Passenger seats: - A seat belt; - A seat belt with UTR in the case of aeroplanes with an MCTOM ≤5 700 kg and with an MOPSC ≤ 9, having an individual CofA first issued on or after 08/04/15. - A child restraint device (CRD) for each person on board younger than 24 months. - Cabin crew seats: - A seat belt with upper torso restraint system on each seat for the minimum required CC. | - See criteria related to CRD, UTR and CC seats in AMCs. |  |
| CAT.IDE.A.210 - Fasten seat belt and no smoking signs ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Aeroplanes in which not all passenger seats are visible from the FC seat(s) shall be equipped with a means of indicating to all passengers and cabin crew when seat belts shall be fastened and when smoking is not allowed. |  |  |
| CAT.IDE.A.215 - Internal doors and curtains ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| ORO.SEC.100 - Applicable to aeroplanes with an MOPSC > 19: - A door between the passenger compartment and the flight crew compartment, with a placard indicating ‘crew only’ and a locking means to prevent passengers from opening it without the permission of a member of the FC. - Applicable to aeroplanes with: - an MCTOM >54.500 kg - an MCTOM > 45.500 kg and an MOPSC > 19; - an MOPSC > 60. An approved secure flight crew compartment door: - that is capable of being locked and unlocked from either pilot's station - with means by which the CC can notify the FC in the event of suspicious activity or security breaches in the cabin; - with means for monitoring from either pilot's station the entire door area outside the FC compartment to identify persons that request to enter and to detect suspicious behaviour or potential threat. Internal doors and curtains: - To be available: - a readily accessible means for opening each door that separates a passenger compartment from another compartment that has emergency exits; - a means for securing in the open position any doorway or curtain separating the passenger compartment from other areas that need to be accessed to reach any required emergency exit from any passenger seat; - a placard on each internal door or adjacent to a curtain that is the means of access to a passenger emergency exit, to indicate that it shall be secured open during take-off and landing; and - a means for any member of the crew to unlock any door that is normally accessible to passengers and that can be locked by passengers. |  |  |
| CAT.IDE.A.220 -First-aid kit (FAK) ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Readily accessible and kept up to date FAKs to be available. - Number of FAK depending on the number of passenger seats: - 0-100: 1 - 101-200: 2 - 201-300: 3 - 301-400: 4 - 401-500: 5 - 501+ : 6 - FAK to be inspected periodically and replenished accordingly | - Check the content of the FAK against AMC1 CAT.IDE.A.220 and the validity of the medications contained. Check in particular the presence of the additional equipment mentioned in para b)4) of AMC1 CAT.IDE.A.220 (AED, bag-valve masks, airway management device,…) - Check that regular checks of the content of the FAK is performed (records). |  |
| CAT.IDE.A.225- Emergency medical kit (EMK) ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Emergency medical kit (EMK): - Applicable to aeroplanes with an MOPSC > 30 and when any point on the planned route is more than 60 minutes flying time at normal cruising speed from an aerodrome at which qualified medical assistance could be expected to be available. - EMK to be: - dust and moisture proof; - carried in a way that prevents unauthorised access; and - kept up to date. | - Check the content of the EMK against AMC1 CAT.IDE.A.225 and the validity of the medications contained. Check in particular the recently added items in the AMC such as pulse oximeter and pneumothorax set. - Check that regular checks of the content of the EMK is performed (records). - EMK should be located in the FC compartment or in the cabin in an area which is not intended for the use of passengers (e.g. not in the overhead compartments). |  |
| CAT.IDE.A.230- First-aid oxygen ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Pressurised aeroplanes operated at pressure altitudes above 25 000 ft, in the case of operations for which a cabin crew member is required, shall be equipped with a supply of undiluted oxygen for passengers who, for physiological reasons, might require oxygen following a cabin depressurisation. -The oxygen supply referred to in (a) shall be sufficient for the remainder of the flight after cabin depressurisation when the cabin altitude exceeds 8 000 ft but does not exceed 15 000 ft, for at least 2 % of the passengers carried, but in no case for less than one person. -There shall be a sufficient number of dispensing units, but in no case less than two, with a means for cabin crew to use the supply. -The first-aid oxygen equipment shall be capable of generating a mass flow to each person. |  |  |
| CAT.IDE.A.235 - Supplemental oxygen – pressurised aeroplanes ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Pressurised aeroplanes operated at pressure altitudes above 10 000 ft shall be equipped with supplemental oxygen equipment that is capable of storing and dispensing the oxygen supplies in accordance with Table. |  |  |
| -Pressurised aeroplanes operated at pressure altitudes above 25 000 ft shall be equipped with: - quick donning types of masks for flight crew members; - sufficient spare outlets and masks or portable oxygen units with masks distributed evenly throughout the passenger compartment, to ensure immediate availability of oxygen for use by each required cabin crew member; - an oxygen dispensing unit connected to oxygen supply terminals immediately available to each cabin crew member, additional crew member and occupants of passenger seats, wherever seated; and - a device to provide a warning indication to the flight crew of any loss of pressurization. |  |  |
| -In the case of pressurised aeroplanes first issued with an individual CofA after 8 November 1998 and operated at pressure altitudes above 25 000 ft, or operated at pressure altitudes at, or below 25 000 ft under conditions that would not allow them to descend safely to 13 000 ft within four minutes, the individual oxygen dispensing units an oxygen dispensing unit connected to oxygen supply terminals immediately available to each cabin crew member, additional crew member and occupants of passenger seats, wherever seated shall be automatically deployable. |  |  |
| -The total number of dispensing units and outlets referred to in (b)(3) and (c) shall exceed the number of seats by at least 10 %. The extra units shall be evenly distributed throughout the passenger compartment. |  |  |
| -Notwithstanding (a), the oxygen supply requirements for cabin crew member(s), additional crew member(s) and passenger(s), in the case of aeroplanes not certified to fly at altitudes above 25 000 ft, may be reduced to the entire flying time between 10 000 ft and 13 000 ft cabin pressure altitudes for all required cabin crew members and for at least 10 % of the passengers if, at all points along the route to be flown, the aeroplane is able to descend safely within four minutes to a cabin pressure altitude of 13 000 ft. |  |  |
| -The required minimum supply in Table above, row 1 item (b)(1) and row 2, shall cover the quantity of oxygen necessary for a constant rate of descent from the aeroplane’s maximum certified operating altitude to 10 000 ft in 10 minutes and followed by 20 minutes at 10 000 ft. |  |  |
| -The required minimum supply in Table above, row 1 item 1(b)(2), shall cover the quantity of oxygen necessary for a constant rate of descent from the aeroplane’s maximum certified operating altitude to 10 000 ft in 10 minutes followed by 110 minutes at 10 000 ft. |  |  |
| -The required minimum supply in Table above, row 3, shall cover the quantity of oxygen necessary for a constant rate of descent from the aeroplane’s maximum certified operating altitude to 15 000 ft in 10 minutes |  |  |
| CAT.IDE.A.240- Supplemental oxygen — non-pressurised aeroplanes ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Supplemental oxygen in accordance with table 1 of CAT.IDE.A.240 applicable to aeroplanes operated above 10.000 ft. |  |  |
| CAT.IDE.A.245 - Crew protective breathing equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - For all pressurised aeroplanes and those unpressurised aeroplanes with an MCTOM > 5 700 kg or having an MOPSC > 19 seats a PBE to protect the eyes, nose and mouth and to provide for a period of at least 15mn: - Oxygen for each FC member on duty in the FC compartment; - Breathing gas from a portable PBE for one member of the FC, adjacent to his/her assigned station, - in the case of aeroplanes operated with a FC of more than one and no CC member. - A PBE intended for FC use to be installed in the FC compartment and be accessible for immediate use by each required FC member at his/her assigned station. - A PBE while in use shall not prevent the use of the required means of communication. Cabin crew: - For all pressurised aeroplanes and those unpressurised aeroplanes with an MCTOM > 5 700 kg or having an MOPSC > 19 seats: A PBE intended for CC use to be installed adjacent to each required CC member station. A PBE to protect the eyes, nose and mouth and to provide for a period of at least 15mn breathing gas for each required CC, adjacent to his/her assigned station; - Additional portable PBE to be installed adjacent to the hand fire extinguisher referred to in points CAT.IDE.A.250 (b) and (c), or adjacent to the entrance of the cargo compartment, in case the hand fire extinguisher is installed in a cargo compartment. - A PBE while in use shall not prevent the use of the required means of communication. | Aeroplanes with an MCTOM of more than 5 700 kg or having an MOPSC of more than 19 |  |
| CAT.IDE.A.250 - Hand fire extinguishers ☐ N/A ☐ Compliant ☐ Non Compliant - Reg. (EU) 2015/640 (Part-26 as amended by Reg. (EU) 2019/133 |  |  |
| - At least one hand fire extinguisher in the FC compartment, suitable for fighting both flammable fluid and electrical equipment fires. - Dry chemical fire extinguishers not to be used in the FC Compartment. Hand fire extinguishers (HFE): - At least one HFE located in, or readily accessible for use in, each galley not located on the main passenger compartment. - At least one HFE shall be available for use in each class A or class B cargo or baggage compartment and in each class E cargo compartment that is accessible to crew members in flight. - Minimum number of HFE depending on MOPSC: - 7-30: 1 - 31-60: 2 - 61-200: 3 - 201-300: 4 - 301-400: 5 - 401-500: 6 - 501-600: 7 - 601+: 8 - Location of HFE to be indicated by a sign. Hand fire extinguishers: - At least one hand fire extinguisher to be available for use in each class A or class B cargo or baggage compartment and in each class E cargo compartment that is accessible to crew members in flight. | - No portable fire extinguisher using halon in large aeroplanes for which the first individual certificate of airworthiness is issued on or after 18 May 2019. |  |
| CAT.IDE.A.255 - Crash axe and crowbar ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - For aeroplanes with an MCTOM > 5 700 kg or with an MOPSC > 9, at least one crash axe or crowbar located in the FC compartment. - For aeroplanes with an MOPSC > 200, an additional crash axe or crowbar to be installed in or near the rearmost galley area, and not visible to passengers. |  |  |
| CAT.IDE.A.260 - Marking of break-in points ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Not required to have such markings. - If such areas are marked, the marking shall follow the criteria of CAT.IDE.A.260 + AMC. |  |  |
| CAT.IDE.A.265 - Means for emergency evacuation ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Aeroplanes with passenger emergency exit sill heights of more than 1,83 m (6 ft) above the ground shall be equipped at each of those exits with a means to enable passengers and crew to reach the ground safely in an emergency. - Notwithstanding (a), such means are not required at overwing exits if the designated place on the aeroplane structure at which the escape route terminates is less than 1,83 m (6 ft) from the ground with the aeroplane on the ground, the landing gear extended, and the flaps in the take-off or landing position, whichever flap position is higher from the ground - Aeroplanes required to have a separate emergency exit for the FC for which the lowest point of the emergency exit is more than 1,83 m (6 ft) above the ground shall have a means to assist all flight crew members in descending to reach the ground safely in an emergency. | - Not required at overwing exits if the designated place on the aeroplane structure at which the escape route terminates is less than 1,83 m (6 ft) from the ground |  |
| CAT.IDE.A.270 - Megaphones ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - For aeroplanes with an MOPSC > 60, megaphone(s) to be available in accordance with the following: - 61-99: 1 - 100+ : 2 - Megaphone(s) to be readily accessible at the assigned seat of a CC, but not necessarily reachable while strapped in CC’s seat. |  |  |
| CAT.IDE.A.275 - Emergency lighting and marking ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - For aeroplanes with an MOPSC ≤ 9, source of general cabin illumination for night operations. - For aeroplanes with an MOPSC > 9, emergency lighting system (with independent power supply) required - For aeroplanes with an MOPSC > 19: - Source of general cabin illumination; - Internal lighting in floor level emergency areas; - Illuminated emergency exit marking and lighting; - When operated by night, exterior emergency lighting at all passenger emergency exits (additional criteria in IR); - A floor proximity emergency escape path marking. NOTE: See additional criteria in IR in particular for aeroplanes with with an MOPSC ≤ 19 | Note: some specific conditions are applicable for aircraft with a TC or equivalent filed before 31/12/1957 or 01/05/1972 (see CAT.IDE.A.275) |  |
| CAT.IDE.A.280 - Emergency locator transmitter (ELT) ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Aeroplane with an MOPSC > 19, at least: - If first issued with an individual CofA after 01/07/08, 2 ELTs (including one automatic), or one ELT and one aircraft localisation means (CAT.GEN.MPA.210); - If first issued with an individual CofA on or before 01/07/08, 1 automatic ELT or 2 ELTs of any type or one aircraft localisation means (CAT.GEN.MPA.210). - Aeroplane with an MOPSC ≤ 19, at least: - If first issued with an individual CofA after 01/07/08, 1 automatic ELT or one aircraft localisation means (CAT.GEN.MPA.210); - If first issued with an individual CofA on or before 01/07/08, 1 ELT of any type or one aircraft localisation means (CAT.GEN.MPA.210). | - Perform when possible an ELT self-test (dedicated button on the FC panel) during the aircraft inspection. - Check that the ELT(s) on-board is/are of one of the types described in the AMC2 and meets the specific criteria of AMC2.. |  |
| CAT.IDE.A.285 - Flight over water ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Life-jackets (with means of electrical illumination) for each person on board, required for: - Landplanes operated over water at a distance > 50 NM from shore or with T-O or LDG path over water; - Seaplanes operated over water. - In addition, seaplanes operated over water, to be equipped with: - A sea anchor and other equipment necessary to facilitate mooring, anchoring or manoeuvring the seaplane on water, appropriate to its size, mass and handling characteristics; - Equipment for making sound signal iaw related collision prevention regulation. - Aeroplanes operated over water at a distance away from land suitable for making an emergency landing, greater than that corresponding to: - 120 minutes at cruising speed or 400 NM, whichever is the lesser, in the case of aeroplanes capable of continuing the flight to an aerodrome with the critical engine(s) becoming inoperative at any point along the route or planned diversions; or - for all other aeroplanes, 30 minutes at cruising speed or 100 NM, whichever is the lesser, to be equipped with: - Lift rafts (accommodation all person on board); - A survivor locator light in each raft; - Adequate life-saving equipment; - 2 survival ELTs. - For aeroplanes with an MCTOM > 27 000 kg and with an MOPSC > 19 and all aeroplanes with an MCTOM > 45 500 kg, a securely attached underwater locating device that operates at a frequency of 8,8 kHz ± 1 kHz, unless: - the aeroplane is operated over routes on which it is at no point at a distance of more than 180 NM from the shore; or - the aeroplane is equipped with robust and automatic means to accurately determine, following an accident where the aeroplane is severely damaged, the location of the point of end of flight. | CAT.GEN.MPA.210 |  |
| CAT.IDE.A.305 - Survival equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Aeroplanes operated over areas in which search and rescue would be especially difficult to be equipped with: - Signalling equipment to make the distress signals; - At least one ELT(S); and - Additional survival equipment (see AMC) for the route to be flown taking account of the number of persons on board. | - The additional survival equipment not needed when the aeroplane: (1) remains within a distance from an area where search and rescue is not especially difficult corresponding to: (i) 120 minutes at OEI cruising speed for aeroplanes capable of continuing the flight to an aerodrome with the critical engine(s) becoming inoperative at any point along the route or planned diversion routes; or (ii) 30 minutes at cruising speed for all other aeroplanes; (2) remains within a distance no greater than that corresponding to 90 minutes at cruising speed from an area suitable for making an emergency landing, for aeroplanes certified in accordance with the applicable airworthiness standard. |  |
| CAT.IDE.A.325 - Headset ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Headset with a boom or throat microphone for each FC member. - For IFR/might operations, a transmit button on the manual pitch and roll control for each required FC member. |  |  |
| CAT.IDE.A.330 - Radio communication equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| As required by the applicable airspace requirements. |  |  |
| CAT.IDE.A.335 - Audio selector panel ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| For IFR, operations an audio selector panel operable from each required FC member station. |  |  |
| CAT.IDE.A.340 - Radio equipment for ops under VFR over routes navigated by reference to visual landmarks ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Aeroplanes operated under VFR over routes navigated by reference to visual landmarks to be equipped with radio communication equipment necessary under normal radio propagation conditions allowing to: - Communicate with appropriate ground stations; - Communicate with appropriate ATC stations from any point in controlled airspace within which flights are intended; and - Receive meteorological information. |  |  |
| CAT.IDE.A.345 - Coms, nav and surveillance eq. for ops under IFR or under VFR over routes not navigated by reference to visual landmarks ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - As required by the applicable airspace requirements (see AMC/GM). - To include at least: - 2 independent radio communication systems; - 1 LR communication system for aeroplanes operated for short haul operations in NAT HLA airspace and not crossing the North Atlantic, in case alternative communication procedures are published for the airspace concerned. |  |  |
| CAT.IDE.A.350 -Transponder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| A pressure altitude reporting secondary surveillance radar (SSR) transponder and any other SSR transponder capability required for the route being flown. |  |  |
| CAT.IDE.A.355 - Management of aeronautical databases ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - For aeronautical databases supporting an airborne navigation application as a primary means of navigation used to meet the airspace usage requirements, database provider to be a Type 2 DAT provider certified in accordance with Regulation (EU) 2017/373 or equivalent. | - Perform a verification of the database validity, if possible, during the aircraft inspection. |  |

| Declaração de conformidade Compliance declaration Statement Eu, _______________________________________________ declaro, sob minha responsabilidade, que o documento por mim submetido cumpre os requisitos nele inscritos e que a aeronave está conforme o aplicável no Regulamento (EU) No 965/2012 e que a informação prestada é verdadeira, completa e precisa. I, hereby declare, under my sole responsibility, that the document I have submitted complies with the requirements set out therein, and that the aircraft complies with the applicable requirements of Regulation (EU) No 965/2012. I further declare that the information provided is true, complete and accurate |  |  |  |
| --- | --- | --- | --- |
| Nome: Name |  | Organização: Organisation |  |
| Função: Position |  | Referência de Aprovação: Approval Reference |  |
|  |  | Assinatura e data Signature and date |  |
| Reservado aos serviços da ANAC Reserved to ANAC services |  |  |  |
| Parecer técnico ANAC : Technical Opinion – ANAC A declaração acima referida cumpre com o requerido pelo Regulamento (EU) No 965/2012: The aforementioned declaration complies with the requirements of Regulation (EU) No 965/20122 ☐ – Integralmente; ☐ – Com exceção dos pontos: ________________________________________________ ☐ – Não cumpre |  |  |  |
| Técnico(s) | Chefe de Departamento |  | Diretor |

ANEXO II Helicópteros

ANEXO II  Helicopters

| Requisito1 Requirement | Requisito Específico2 Specific Requirement | Método de cumprimento ou razão para não cumprimento-Operador3 Method of compliance or Reason for Non applicability-Operator |
| --- | --- | --- |
| CAT.IDE.H.100 Instruments and equipment – general ☐ N/A ☐ Compliant ☐ Non-Compliant |  |  |
| - Single instrument (required to be installed in a helicopter operated in a multi-crew environment), to be readily operable and visible from each flight crew station. - All required emergency equipment to be easily accessible for immediate use. |  |  |
| CAT.IDE.H.105 Minimum equipment for flight ☐ N/A ☐ Compliant ☐ Non-Compliant |  |  |
| - the helicopter is operated in accordance with the operator’s MEL; or - the operator is approved by the competent authority to operate the helicopter within the constraints of the MMEL in accordance with point ORO.MLR.105(j) of Annex III. |  |  |
| CAT.IDE.H.115 Operating lights ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Helicopters operated under VFR by day shall be equipped with an anti-collision light system. |  |  |
| Helicopters operated at night or under IFR shall, in addition to (a), be equipped with: - lighting supplied from the helicopter’s electrical system to provide adequate illumination for all instruments and equipment essential to the safe operation of the helicopter; - lighting supplied from the helicopter’s electrical system to provide illumination in all passenger compartments; - an independent portable light for each required crew member readily accessible to crew members when seated at their designated stations; - navigation/position lights; - two landing lights of which at least one is adjustable in flight so as to illuminate the ground in front of and below the helicopter and the ground on either side of the helicopter; and - lights to conform with the International Regulations for Preventing Collisions at Sea if the helicopter is amphibious |  |  |
| CAT.IDE.H.125- Operations under VFR by day – flight and navigational instruments and associated equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - A means of measuring and displaying: - Magnetic heading; - Time in hours, minutes and seconds; - Indicated airspeed; - Vertical speed; - Slip; - Outside air temperature (in °C); and -A means of indicating when the supply of power to the required flight instruments is not adequate. |  |  |
| Whenever two pilots are required for the operation, an additional separate means of displaying the following shall be available for the second pilot: - Barometric altitude; - Indicated airspeed; - Vertical speed; and - Slip. |  |  |
| Helicopters with an MCTOM of more than 3 175 kg or any helicopter operating over water when out of sight of land or when the visibility is less than 1 500 m, shall be equipped with a means of measuring and displaying: - Attitude; and - Heading. |  |  |
| A means for preventing malfunction of the airspeed indicating systems due to condensation or icing shall be available for helicopters with an MCTOM of more than 3 175 kg or an MOPSC of more than nine. |  |  |
| CAT.IDE.H.130 Operations under IFR or at night – flight and navigational instruments and associated equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -A means of measuring and displaying: - Magnetic heading; - Time in hours, minutes and seconds; - Indicated airspeed; - Vertical speed; - Slip, - Attitude; - Stabilised heading; - Outside air temperature; -Two means of measuring and displaying barometric altitude. -A means of indicating when the supply of power to the required flight instruments is not adequate. -A means for preventing malfunction of the airspeed indicating systems required due to condensation or icing. - Means of annunciating to the flight crew the failure of the means required in (d) for helicopters: - issued with an individual CofA on or after 1 August 1999; or - issued with an individual CofA before 1 August 1999 with an MCTOM of more than 3 175 kg, and with an MOPSC of more than nine |  |  |
| A standby means of measuring and displaying attitude that: - is powered continuously during normal operation and, in the event of a total failure of the normal electrical generating system, is powered from a source independent of the normal electrical generating system; - operates independently of any other means of measuring and displaying attitude; - is capable of being used from either pilot’s station; - is operative automatically after total failure of the normal electrical generating system; - provides reliable operation for a minimum of 30 minutes or the time required to fly to a suitable alternate landing site when operating over hostile terrain or offshore, whichever is greater, after total failure of the normal electrical generating system, taking into account other loads on the emergency power supply and operational procedures; - is appropriately illuminated during all phases of operation; and - is associated with a means to alert the flight crew when operating under its dedicated power supply, including when operated by emergency power. |  |  |
| -An alternate source of static pressure for the means of measuring altitude, airspeed and vertical speed. |  |  |
| Whenever two pilots are required for the operation, a separate means for displaying for the second pilot: - Barometric altitude - Indicated airspeed; - Vertical speed; - Slip; - Attitude; and - Stabilised heading. |  |  |
| -For IFR operations, a chart holder in an easily readable position that can be illuminated for night operations. |  |  |
| CAT.IDE.H.135- Additional equipment for single-pilot operation under IFR ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Helicopters operated under IFR with a single-pilot shall be equipped with an autopilot with at least - altitude hold and heading mode. |  |  |
| CAT.IDE.H.160 Airborne weather detecting equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Helicopters with an MOPSC of more than nine and operated under IFR or at night shall be equipped with airborne weather detecting equipment when current weather reports indicate that thunderstorms or other potentially hazardous weather conditions, regarded as detectable with airborne weather detecting equipment, may be expected to exist along the route to be flown |  |  |
| CAT.IDE.H.165 - Additional equipment for operations in icing conditions at night ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters operated in expected or actual icing conditions at night shall be equipped with a means to illuminate or detect the formation of ice. -The means to illuminate the formation of ice shall not cause glare or reflection that would handicap crew members in the performance of their duties. |  |  |
| CAT.IDE.H.170 - Flight crew interphone system ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Helicopters operated by more than one flight crew member shall be equipped with a flight crew interphone system, including headsets and microphones for use by all flight crew members. |  |  |
| CAT.IDE.H.175 - Crew member interphone system ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Helicopters shall be equipped with a crew member interphone system when carrying a crew member other than a flight crew member. |  |  |
| CAT.IDE.H.180- Public address system ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -with an MOPSC of more than nine shall be equipped with a public address system, with the exception of the following paragraph -Notwithstanding helicopters with an MOPSC of more than nine and less than 20 are exempted from having a public address system, if: - the helicopter is designed without a bulkhead between pilot and passengers; and - the operator is able to demonstrate that when in flight, the pilot’s voice is audible and intelligible at all passengers’ seats. |  |  |
| CAT.IDE.H.185 - Cockpit voice recorder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -The following helicopter types shall be equipped with a cockpit voice recorder (CVR): - all helicopters with an MCTOM of more than 7 000 kg; and - helicopters with an MCTOM of more than 3 175 kg and first issued with an individual CofA on or after 1 January 1987. |  |  |
| -The CVR shall be capable of retaining the data recorded during at least: - the preceding two hours for helicopters referred to in (a)(1) and (a)(2), when first issued with an individual CofA on or after 1 January 2016; - the preceding one hour for helicopters referred to in (a)(1), when first issued with an individual CofA on or after 1 August 1999 and before 1 January 2016; - the preceding 30 minutes for helicopters referred to in (a)(1), when first issued with an individual CofA before 1 August 1999; or - the preceding 30 minutes for helicopters referred to in (a)(2), when first issued with an individual CofA before 1 January 2016. |  |  |
| By 1 January 2019 at the latest, the CVR shall record on means other than magnetic tape or magnetic wire |  |  |
| The CVR shall record with reference to a timescale -voice communications transmitted from or received in the flight crew compartment by radio; -flight crew members' voice communications using the interphone system and the public address system, if installed; - the aural environment of the flight crew compartment, including without interruption: - for helicopters first issued with an individual CofA on or after 1 August 1999, the audio signals received from each crew microphone; - for helicopters first issued with an individual CofA before 1 August 1999, the audio signals received from each crew microphone, where practicable; -voice or audio signals identifying navigation or approach aids introduced into a headset or speaker. |  |  |
| -The CVR shall start to record prior to the helicopter moving under its own power and shall continue to record until the termination of the flight when the helicopter is no longer capable of moving under its own power. -In addition, for helicopters referred to MCTOM of more than 3 175 kg issued with an individual CofA on or after 1 August 1999: - the CVR shall start automatically to record prior to the helicopter moving under its own power and continue to record until the termination of the flight when the helicopter is no longer capable of moving under its own power; and depending on the availability of electrical power, the CVR shall start to record as early as possible during the cockpit checks prior to engine start at the beginning of the flight until the cockpit checks immediately following engine shutdown at the end of the flight. |  |  |
| -If the CVR is not deployable, it shall have a device to assist in locating it under water. By 1 January 2020 at the latest, this device shall have a minimum underwater transmission time of 90 days. If the CVR is deployable, it shall have an automatic emergency locator transmitter. |  |  |
| CAT.IDE.H.190 - Flight data recorder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - The following helicopters shall be equipped with an FDR that uses a digital method of recording and storing data and for which a method of readily retrieving that data from the storage medium is available: - helicopters with an MCTOM of more than 3 175 kg and first issued with an individual CofA on or after 1 August 1999; - helicopters with an MCTOM of more than 7 000 kg, or an MOPSC of more than nine, and first issued with an individual CofA on or after 1 January 1989 but before 1 August 1999 |  |  |
| - The FDR shall record the parameters required to determine accurately the: - flight path, speed, attitude, engine power, operation and configuration and be capable of retaining the data recorded during at least the preceding 10 hours, for helicopters referred to in (a)(1) and first issued with an individual CofA on or after 1 January 2016; - flight path, speed, attitude, engine power and operation and be capable of retaining the data recorded during at least the preceding eight hours, for helicopters referred to in (a)(1) and first issued with an individual CofA before 1 January 2016; - flight path, speed, attitude, engine power and operation and be capable of retaining the data recorded during at least the preceding five hours, for helicopters referred to in (a)(2). |  |  |
| -Data shall be obtained from helicopter sources that enable accurate correlation with information displayed to the flight crew. -The FDR shall automatically start to record the data prior to the helicopter being capable of moving under its own power and shall stop automatically after the helicopter is incapable of moving under its own power. -If the FDR is not deployable, it shall have a device to assist in locating it under water. By 1 January 2020 at the latest, this device shall have a minimum underwater transmission time of 90 days. If the FDR is deployable, it shall have an automatic emergency locator transmitter. |  |  |
| CAT.IDE.H.191 Lightweight flight recorder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Turbine-engined helicopters with an MCTOM of 2 250 kg or more shall be equipped with a flight recorder if all of the following conditions are met: - they are not within the scope of point CAT.IDE.H.190(a); - they are first issued with an individual CofA on or after 5 September 2022. -The flight recorder shall record, by means of flight data or images, information that is sufficient to determine the flight path and aircraft speed. -The flight recorder shall be capable of retaining the flight data and the images recorded during at least the preceding 5 hours. -The flight recorder shall automatically start to record prior to the helicopter being capable of moving under its own power and shall stop automatically after the helicopter is no longer capable of moving under its own power. -If the flight recorder records images or audio of the flight crew compartment, then a function shall be provided which can be operated by the commander and which modifies image and audio recordings made before the operation of that function, so that those recordings cannot be retrieved using normal replay or copying techniques. |  |  |
| CAT.IDE.H.195 - Data link recording ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Helicopters first issued with an individual CofA on or after 8 April 2014 that have the capability to operate data link communications and are required to be equipped with a CVR, shall record on a recorder, where applicable: - data link communication messages related to ATS communications to and from the helicopter, including messages applying to the following applications: - data link initiation; - controller-pilot communication; - addressed surveillance; - flight information; - as far as is practicable, given the architecture of the system, aircraft broadcast surveillance; - as far as is practicable, given the architecture of the system, aircraft operational control data; - as far as is practicable, given the architecture of the system, graphics; - information that enables correlation to any associated records related to data link communications and stored separately from the helicopter; and - information on the time and priority of data link communications messages, taking into account the system’s architecture. - The recorder shall use a digital method of recording and storing data and information and a method of readily retrieving that data shall be available. The recording method shall allow the data to match the data recorded on the ground. - The recorder shall be capable of retaining data recorded for at least the same duration as set out for CVRs in CAT.IDE.H.185. |  |  |
| CAT.IDE.H.200 Flight data and cockpit voice combination recorder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Compliance with CVR and FDR requirements may be achieved by the carriage of one combination recorder. |  |  |
| CAT.IDE.H.205 - Seats, seat safety belts, restraint systems and child restraint devices ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters shall be equipped with: - a seat or berth for each person on board who is aged 24 months or more; - a seat belt on each passenger seat and restraining belts for each berth; - For helicopters first issued with an individual CofA on or after 1 August 1999, a safety belt with upper torso restraint system for use on each passenger seat for each passenger aged 24 months or more; - a child restraint device (CRD) for each person on board younger than 24 months; - a seat belt with upper torso restraint system incorporating a device that will automatically restrain the occupant’s torso in the event of rapid deceleration on each flight crew seat; - a seat belt with upper torso restraint system on each seat for the minimum required cabin crew. -A seat belt with upper torso restraint system shall: - have a single point release; and - on flight crew seats and on the seats for the minimum required cabin crew include two shoulder straps and a seat belt that may be used independently. | - See criteria related to CRD, UTR and CC seats in AMCs. |  |
| CAT.IDE.H.210 - Fasten seat belt and no smoking signs ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Helicopters in which not all passenger seats are visible from the flight crew seat(s) shall be equipped with a means of indicating to all passengers and cabin crew when seat belts shall be fastened and when smoking is not allowed. |  |  |
| CAT.IDE.H.220 -First-aid kit (FAK) ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters shall be equipped with at least one first-aid kit. -First-aid kits shall be: - readily accessible for use; - kept up to date. |  |  |
| CAT.IDE.H.240 Supplemental oxygen – non-pressurised helicopters ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Non-pressurised helicopters operated at pressure altitudes above 10 000 ft shall be equipped with supplemental oxygen equipment capable of storing and dispensing the oxygen supplies inaccordance with the following tables. |  |  |
| CAT.IDE.H.250 Hand fire extinguishers ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters shall be equipped with at least one hand fire extinguisher in the flight crew compartment. -At least one hand fire extinguisher shall be located in, or readily accessible for use in, each galley not located on the main passenger compartment. - At least one hand fire extinguisher shall be available for use in each cargo compartment that is accessible to crew members in flight. -The type and quantity of extinguishing agent for the required fire extinguishers shall be suitable for the type of fire likely to occur in the compartment where the extinguisher is intended to be used and to minimise the hazard of toxic gas concentration in compartments occupied by persons. -The helicopter shall be equipped with at least a number of hand fire extinguishers - 7-30: 1 - 31-60: 2 - 61-200: 3 |  |  |
| CAT.IDE.H.260 - Marking of break-in points ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| If areas of the helicopter’s fuselage suitable for break-in by rescue crews in an emergency are marked, such areas shall be marked as shown CAT.IDE.H.260 |  |  |
| CAT.IDE.H.270 - Megaphones ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Helicopters with an MOPSC of more than 19 shall be equipped with one portable battery-powered megaphone readily accessible for use by crew members during an emergency evacuation. |  |  |
| CAT.IDE.H.275 - Emergency lighting and marking ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters with an MOPSC of more than 19 shall be equipped with: - an emergency lighting system having an independent power supply to provide a source of general cabin illumination to facilitate the evacuation of the helicopter; and - emergency exit marking and locating signs visible in daylight or in the dark. |  |  |
| -Helicopters shall be equipped with emergency exit markings visible in daylight or in the dark when operated: - in performance class 1 or 2 on a flight over water at a distance from land corresponding to more than 10 minutes flying time at normal cruising speed; - in performance class 3 on a flight over water at a distance corresponding to more than three minutes flying time at normal cruising speed. |  |  |
| CAT.IDE.H.280 - Emergency locator transmitter (ELT) ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters shall be equipped with at least one automatic ELT. -An ELT of any type shall be capable of transmitting simultaneously on 121.5 MHz and 406 MHz. |  |  |
| CAT.IDE.H.290 Life-jackets ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters shall be equipped with a life-jacket for each person on board or equivalent floatation device for each person on board younger than 24 months, stowed in a position that is readily accessible from the seat or berth of the person for whose use it is provided, when operated in: - performance class 1 or 2 on a flight over water at a distance from land corresponding to more than 10 minutes flying time at normal cruising speed; - performance class 3 on a flight over water beyond autorotational distance from land; - performance class 2 or 3 when taking off or landing at an aerodrome or operating site where the take-off or approach path is over water. |  |  |
| CAT.IDE.H.295 Crew survival suits ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Each crew member shall wear a survival suit when operating in performance class 3 on a flight over water beyond autorotational distance or safe forced landing distance from land, when the weather report or forecasts available to the commander indicate that the sea temperature will be less than plus 10 °C during the flight. |  |  |
| CAT.IDE.H.300 Life-rafts, survival ELTs and survival equipment on extended overwater flights ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - in performance class 1 or 2 on a flight over water at a distance from land corresponding to more than 10 minutes flying time at normal cruising speed; - in performance class 3 on a flight over water at a distance corresponding to more than three minutes flying time at normal cruising speed, shall be equipped with: - in the case of a helicopter carrying less than 12 persons, at least one life-raft with a rated capacity of not less than the maximum number of persons on board, stowed so as to facilitate its ready use in an emergency; - in the case of a helicopter carrying more than 11 persons, at least two life-rafts, stowed so as to facilitate their ready use in an emergency, sufficient together to accommodate all persons capable of being carried on board and, if one is lost, the remaining life-raft(s) having, the overload capacity sufficient to accommodate all persons on the helicopter; - at least one survival ELT (ELT(S)) for each required life-raft; and - life-saving equipment, including means of sustaining life, as appropriate to the flight to be undertaken. |  |  |
| CAT.IDE.H.305 Survival equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters operated over areas in which search and rescue would be especially difficult shall be equipped with: - signalling equipment to make distress signals; - at least one ELT(S); and - additional survival equipment for the route to be flown taking account of the number of persons on board. |  |  |
| CAT.IDE.H.315 Helicopters certified for operating on water – miscellaneous equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Helicopters certified for operating on water shall be equipped with: - a sea anchor and other equipment necessary to facilitate mooring, anchoring or manoeuvring the helicopter on water, appropriate to its size, mass and handling characteristics; and - equipment for making the sound signals prescribed in the International Regulations for Preventing Collisions at Sea, where applicable |  |  |
| CAT.IDE.H.320 All helicopters on flights over water – ditching ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters shall be designed for landing on water or certified for ditching in accordance with the relevant certification specification when operated in performance class 1 or 2 on a flight over water in a hostile environment at a distance from land corresponding to more than 10 minutes flying time at normal cruise speed. -Helicopters shall be designed for landing on water or certified for ditching in accordance with the relevant certification specification or fitted with emergency flotation equipment when operated in: - performance class 1 or 2 on a flight over water in a non-hostile environment at a distance from land corresponding to more than 10 minutes flying time at normal cruise speed; - performance class 2, when taking off or landing over water, except in the case of helicopter emergency medical services (HEMS) operations, where for the purpose of minimising exposure, the landing or take-off at a HEMS operating site located in a congested environment is conducted over water; - performance class 3 on a flight over water beyond safe forced landing distance from land. |  |  |
| CAT.IDE.H.325 Headset ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Whenever a radio communication and/or radio navigation system is required, helicopters shall be equipped with a headset with boom microphone or equivalent and a transmit button on the flight controls for each required pilot and/or crew member at his/her assigned station. |  |  |
| CAT.IDE.H.330 Radio communication equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters shall be equipped with the radio communication equipment required by the applicable airspace requirements. -The radio communication equipment shall provide for communication on the aeronautical emergency frequency 121,5 MHz. |  |  |
| CAT.IDE.H.335 Audio selector panel ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Helicopters operated under IFR shall be equipped with an audio selector panel operable from each required flight crew member station. |  |  |
| CAT.IDE.H.340 Radio equipment for operations under VFR over routes navigated by reference to visual landmarks ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -communicate with appropriate ground stations; -communicate with appropriate ATC stations from any point in controlled airspace within which flights are intended; and -receive meteorological information. |  |  |
| CAT.IDE.H.345 Comm, navigation and surv. equipment for operations under IFR or under VFR over routes not navigated by reference to visual landmarks ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters operated under IFR or under VFR over routes that cannot be navigated by reference to visual landmarks shall be equipped with radio communication, navigation and surveillance equipment in accordance with the applicable airspace requirements. -Radio communication equipment shall include at least two independent radio communication systems necessary under normal operating conditions to communicate with an appropriate ground station from any point on the route, including diversions. - Helicopters shall have sufficient navigation equipment to ensure that, in the event of the failure of one item of equipment at any stage of the flight, the remaining equipment shall allow safe navigation in accordance with the flight plan. - Helicopters operated on flights in which it is intended to land in IMC shall be equipped with suitable equipment capable of providing guidance to a point from which a visual landing can be performed for each aerodrome at which it is intended to land in IMC and for any designated alternate aerodromes. - For PBN operations the aircraft shall meet the airworthiness certification requirements for the appropriate navigation specification. |  |  |
| CAT.IDE.H.350 Transponder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Helicopters shall be equipped with a pressure altitude reporting secondary surveillance radar (SSR) transponder and any other SSR transponder capability required for the route being flown. |  |  |
| CAT.IDE.H.355 Management of aeronautical databases ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Aeronautical databases used on certified aircraft system applications shall meet data quality requirements that are adequate for the intended use of the data. - The operator shall ensure the timely distribution and insertion of current and unaltered aeronautical databases to all aircraft that require them. - Notwithstanding any other occurrence reporting requirements as defined in Regulation (EU) No 376/2014, the operator shall report to the database provider instances of erroneous, inconsistent or missing data that might be reasonably expected to constitute a hazard to flight. In such cases, the operator shall inform flight crew and other personnel concerned, and shall ensure that the affected data is not used |  |  |

| Declaração de conformidade Compliance declaration Statement Eu, _______________________________________________ declaro, sob minha responsabilidade, que o documento por mim submetido cumpre os requisitos nele inscritos e que a aeronave está conforme o aplicável no Regulamento (EU) No 965/2012 e que a informação prestada é verdadeira, completa e precisa. I, hereby declare, under my sole responsibility, that the document I have submitted complies with the requirements set out therein, and that the aircraft complies with the applicable requirements of Regulation (EU) No 965/2012. I further declare that the information provided is true, complete and accurate |  |  |  |
| --- | --- | --- | --- |
| Nome: Name |  | Organização: Organisation |  |
| Função: Position |  | Referência de Aprovação: Approval Reference |  |
|  |  | Assinatura e data Signature and date |  |
| Reservado aos serviços da ANAC Reserved to ANAC services |  |  |  |
| Parecer técnico ANAC : Technical Opinion – ANAC A declaração acima referida cumpre com o requerido pelo Regulamento (EU) No 965/2012: The aforementioned declaration complies with the requirements of Regulation (EU) No 965/2012 ☐ – Integralmente; ☐ – Com exceção dos pontos: ________________________________________________ ☐ – Não cumpre |  |  |  |
| Técnico(s) | Chefe de Departamento |  | Diretor |