| Declaração de Conformidade EASA-OPS Part SPO.IDE Statement of Compliance with EASA-OPS Part SPO.IDE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operador Operator |  | Registo Registration |  | Modelo Aeronave Aircraft Model |  | Número série Serial Number | Data fabrico Manufacture date |
| MCTOM Maximum Certified Take-off Mass |  | MOPSC Maximum Operational Passenger Seating Configuration |  | Data do 1º CofA Date of 1ª Cofa |  | TCDS Aeronave Aircraft TCDS | TCDS Motor Engine TCDS |
| Tipo de Operações Type of operations |  |  |  |  |  |  |  |
| IFR ☐ VFR ☐ | Dia/Noite ☐ Day/night Dia ☐ Day |  | Cond. de Gelo ☐ Icing conditions Sem cond. de gelo ☐ Non Icing Cond. |  | Sobre água ☐ Over water Sobre terra ☐ Over land |  | Duplo comando ☐ Dual Command Comando único ☐ Single Pilot |
| Aeronave Aircraft |  |  |  |  |  |  |  |
| Pressurizada ☐ Pressurized Não pressurizada ☐ Non Pressurized |  | Altitude máxima Max. Altitude |  | OAT intervalo OAT interval |  |  |  |
| Outra informação relevante: Other relevant information: |  |  |  |  |  |  |  |
| Documentação que sustenta o pedido Documents that support the request |  |  |  |  |  |  |  |
| Refª Publicação Publication Reference | Descrição ou nome da publicação Description or name of the publication |  |  |  |  |  |  |

# TABELA 1

| Anexo Annex | Relatórios Reports |
| --- | --- |
|  | CVR conforme AMC |
|  | Gravação de dados do FDR conforme AMC Flight data recorder |
|  | Gravação de dados do Data Link conforme AMC Data link recording |
|  | Gravação de dados do Combination Recorder AMC Combination recorder |

Regras de preenchimento

Seguidamente descreve-se como é composto o documento e qual deverá ser a forma de preenchimento do mesmo.

O requerente (operador/proprietário) deve preencher o campo 3 – “Método de cumprimento ou razão para não cumprimento – Operador”, com base nos GM e AMC em vigor. Nos campos   ☐ N/A    ☐ Compliant   ☐ Non Compliant deverá também identificar se cumpre, não é aplicável ou não cumpre o requisito.

No campo 3, o requerente deve indicar, de forma clara:

- o método de cumprimento adotado para cada requisito, ou

- a razão para o não cumprimento, quando aplicável.

Sempre que a justificação do operador/proprietário seja suportada por documentação específica, deverá:

- identificar o documento no campo 3 (por exemplo, título, referência e data), e

- anexar esse documento ao requerimento, de forma a fundamentar o cumprimento do requisito.

Deverá ainda preencher a Declaração de Conformidade, datando e assinando a mesma.

Apenas necessita enviar o documento referente ao Anexo para o qual está a submeter a aplicação, isto é, se for para Helicóptero-Anexo II não necessita submeter o Anexo I referente a Aviões, e vice-versa.

ANEXO I Aviões

ANEXO I Airplanes

| Requisito1 Requirement | Requisito Específico2 Specific Requirement | Método de cumprimento ou razão para não cumprimento-Operador3 Method of compliance or Reason for Non applicability-Operator |
| --- | --- | --- |
| SPO.IDE.A.105 - Minimum equipment for flight ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - the aeroplane is operated in accordance with the minimum equipment list (MEL); - for complex motor-powered aeroplanes and for any aeroplane used in commercial operations, the operator is approved by the competent authority to operate the aeroplane within the constraints of the master minimum equipment list (MMEL) in accordance with point ORO.MLR.105(j) of Annex III; - the aeroplane is subject to a permit to fly issued in accordance with the applicable airworthiness requirements. |  |  |
| SPO.IDE.A.110 - Spare electrical fuses ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Aeroplanes shall be equipped with spare electrical fuses, of the ratings required for complete circuit protection, for replacement of those fuses that are allowed to be replaced in flight. |  |  |
| SPO.IDE.A.115 Operating lights ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Aeroplanes operated at night shall be equipped with: - an anti-collision light system; - navigation/position lights; - a landing light; - lighting supplied from the aeroplane’s electrical system to provide adequate illumination for all instruments and equipment essential to the safe operation of the aeroplane; - lighting supplied from the aeroplane’s electrical system to provide illumination in all cabin compartments; - an independent portable light for each crew member station; and - lights to conform with the International Regulations for Preventing Collisions at Sea if the aeroplane is operated as a seaplane. |  |  |
| SPO.IDE.A.120 - Operations under VFR – flight and navigational instruments and associated equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Aeroplanes operated under VFR by day shall be equipped with a means of measuring and displaying the following: - magnetic heading, - time in hours, minutes and seconds, - barometric altitude, - indicated airspeed, - Mach number whenever speed limitations are expressed in terms of Mach number, and - slip for complex motor-powered aeroplanes. |  |  |
| -Aeroplanes operating under VMC at night shall be, in addition to be, equipped with: - turn and slip, - attitude, - vertical speed, and - stabilised heading; -a means of indicating when the supply of power to the gyroscopic instruments is not adequate. |  |  |
| -Complex motor-powered aeroplanes operating under VMC over water and out of sight of the land shall be, in addition to (a) and (b), equipped with a means of preventing malfunction of the airspeed indicating system due to condensation or icing. |  |  |
| -Aeroplanes operated in conditions where they cannot be maintained in a desired flight path without reference to one or more additional instruments, shall be, in addition to be, equipped with a means of preventing malfunction of the airspeed indicating system required in due to condensation or icing. |  |  |
| Whenever two pilots are required for the operation, aeroplanes shall be equipped with an additional separate means of displaying the following: - barometric altitude, - indicated airspeed, - slip, or turn and slip, as applicable, - attitude, if applicable, - vertical speed, if applicable - stabilised heading, if applicable, and - Mach number whenever speed limitations are expressed in terms of Mach number, if applicable. |  |  |
| SPO.IDE.A.125 Operations under IFR – flight and navigational instruments and associated equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| For operations under IFR shall be equipped with: - magnetic heading, - time in hours, minutes and seconds, - barometric altitude, - indicated airspeed, - vertical speed, - turn and slip, - attitude, - stabilised heading, - outside air temperature, and - Mach number, whenever speed limitations are expressed in terms of Mach number; |  |  |
| -a means of indicating when the supply of power to the gyroscopic instruments is not adequate |  |  |
| -whenever two pilots are required for the operation, an additional separate means of displaying for the second pilot: - barometric altitude, - indicated airspeed, - vertical speed, - turn and slip, - attitude, - stabilised heading, and - Mach number whenever speed limitations are expressed in terms of Mach number, if applicable; |  |  |
| a means of preventing malfunction of the airspeed indicating system required in) due to condensation or icing; |  |  |
| - complex motor-powered aeroplanes when operated under IFR shall, in addition be equipped with: - an alternate source of static pressure; - a chart holder in an easily readable position that can be illuminated for night operations; - a second independent means of measuring and displaying altitude unless already installed - an emergency power supply, independent of the main electrical generating system, for the purpose of operating and illuminating an attitude indicating system for a minimum period of 30 minutes. The emergency power supply shall be automatically operative after the total failure of the main electrical generating system and clear indication shall be given on the instrument or on the instrument panel that the attitude indicator is being operated by emergency power. |  |  |
| SPO.IDE.A.126 - Additional equipment for single-pilot operation under IFR ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Complex motor-powered aeroplanes operated under IFR with a single pilot shall be equipped with an autopilot with at least altitude hold and heading mode. |  |  |
| SPO.IDE.A.130 - Terrain awareness warning system (TAWS) ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Turbine-powered aeroplanes with a maximum certified take-off mass (MCTOM) of more than 5 700 kg or an MOPSC of more than nine shall be equipped with a TAWS that meets the requirements for: - class A equipment, as specified in an acceptable standard, in the case of aeroplanes for which the individual certificate of airworthiness (CofA) was first issued after 1 January 2011; or - class B equipment, as specified in an acceptable standard, in the case of aeroplanes for which the individual CofA was first issued on or before 1 January 2011. |  |  |
| -When used in commercial operations, turbine-powered aeroplanes for which the individual CofA was first issued after 1 January 2019 and having an MCTOM of 5 700 kg or less and an MOPSC of six to nine shall be equipped with a TAWS that meets the requirements for class B equipment, as specified in an acceptable standard. |  |  |
| SPO.IDE.A.131 - Airborne collision avoidance system (ACAS II) ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Unless otherwise provided for by Regulation (EU) No 1332/2011, turbine-powered aeroplanes with an MCTOM of more than 5 700 kg shall be equipped with ACAS II. |  |  |
| SPO.IDE.A.132 - Airborne weather detecting equipment – complex motor-powered aeroplanes ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Airborne weather detecting equipment when operated at night or in IMC in areas where thunderstorms or other potentially hazardous weather conditions, regarded as detectable with airborne weather detecting equipment, may be expected to exist along the route: - pressurised aeroplanes; - non-pressurised aeroplanes with an MCTOM of more than 5 700 kg. |  |  |
| SPO.IDE.A.133 Additional equipment for operations in icing conditions at night – complex motor-powered aeroplanes ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Aeroplanes operated in expected or actual icing conditions at night shall be equipped with a means to illuminate or detect the formation of ice. - The means to illuminate the formation of ice shall not cause glare or reflection that would handicap flight crew members in the performance of their duties. |  |  |
| SPO.IDE.A.135 Flight crew interphone system ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Aeroplanes operated by more than one flight crew member shall be equipped with a flight crew interphone system, including headsets and microphones for use by all flight crew members. |  |  |
| SPO.IDE.A.140 Cockpit voice recorder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Aeroplanes shall be equipped with a CVR: -aeroplanes with an MCTOM of more than 27 000 kg and first issued with an individual CofA on or after 1 January 2016; -aeroplanes with an MCTOM of more than 2 250 kg: - certified for operation with a minimum crew of at least two pilots; - equipped with turbojet engine(s) or more than one turboprop engine; and - for which a type certificate is first issued on or after 1 January 2016. |  |  |
| The CVR shall be capable of retaining data recorded during at least: - the preceding 25 hours for aeroplanes with an MCTOM of more than 27 000 kg and first issued with an individual CofA on or after 1 January 2022; - the preceding 2 hours in all other cases. |  |  |
| The CVR shall record with reference to a timescale: - voice communications transmitted from or received in the flight crew compartment by radio; - address system, if installed; - the aural environment of the flight crew compartment, including, without interruption, the audio signals received from each boom and mask microphone in use; - voice or audio signals identifying navigation or approach aids introduced into a headset or speaker. - The CVR shall start automatically to record prior to the aeroplane moving under its own powerand shall continue to record until the termination of the flight when the aeroplane is no longer capable of moving under its own power. - In addition depending on the availability of electrical power, the CVR shall start to record as early as possible during the cockpit checks prior to engine start at the beginning of the flight until the cockpit checks immediately following engine shutdown at the end of the flight. - If the CVR is not deployable, it shall have a device to assist in locating it under water. By 1 January 2020 at the latest, this device shall have a minimum underwater transmission time of 90 days. If the CVR is deployable, it shall have an automatic emergency locator transmitter. |  |  |
| SPO.IDE.A.145 Flight data recorder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Aeroplanes with an MCTOM of more than 5 700 kg and first issues individual CofA on or after 1 January 2016 shall be equipped with an FDR that uses a digital method of recording and storing data and for which a method of readily retrieving that data from the storage medium is available. - The FDR shall record the parameters required to determine accurately the aeroplane flight path, speed, attitude, engine power, configuration and operation and be capable of retaining data recorded during at least the preceding 25 hours. - Data shall be obtained from aeroplane sources that enable accurate correlation with information displayed to the flight crew. - The FDR shall start automatically to record the data prior to the aeroplane being capable of moving under its own power and shall stop automatically after the aeroplane is incapable of moving under its own power. - If the FDR is not deployable, it shall have a device to assist in locating it under water. By 1 January 2020 at the latest, this device shall have a minimum underwater transmission time of 90 days. - If the FDR is deployable, it shall have an automatic emergency locator transmitter. |  |  |
| SPO.IDE.A.146 Lightweight flight recorder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Turbine-engined aeroplanes with an MCTOM of 2 250 kg or more and aeroplanes with an MOPSC of more than 9 shall be equipped with a flight recorder if all the following conditions are met: - they are not within the scope of point SPO.IDE.A.145(a); - they are used for commercial operations; - they are first issued with an individual CofA on or after 5 September 2022. |  |  |
| -The flight recorder shall record, by means of flight data or images, information that is sufficient to determine the flight path and aircraft speed. -The flight recorder shall be capable of retaining the flight data and the images recorded during at least the preceding 5 hours. -The flight recorder shall automatically start to record prior to the aeroplane being capable of moving under its own power and shall stop automatically after the aeroplane is no longer capable of moving under its own power. - If the flight recorder records images or audio of the flight crew compartment, then a function shall be provided which can be operated by the pilot-in-command and which modifies image and audio recordings made before the operation of that function, so that those recordings cannot be retrieved using normal replay or copying techniques. |  |  |
| SPO.IDE.A.150 Data link recording ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -As a means of compliance the recorder on which the data link messages are recorded may be: - CVR - FDR - A combination recorder when SPO.IDE.A.155 is applicable; -information that enables correlation to any associated records related to data link communications and stored separately from the aeroplane; and -information on the time and priority of data link communications messages, taking into account the system’s architecture. -The recorder shall use a digital method of recording and storing data and information and a method for readily retrieving that data. The recording method shall allow the data to match the data recorded on the ground. -The recorder shall be capable of retaining data recorded for at least the same duration as set out for CVRs in SPO.IDE.A.140. -If the recorder is not deployable, it shall have a device to assist in locating it under water. By 1 January 2020 at the latest, this device shall have a minimum underwater transmission time of 90 days. If the recorder is deployable, it shall have an automatic emergency locator transmitter. -The requirements applicable to the start and stop logic of the recorder are the same as the requirements applicable to the start and stop logic of the CVR contained in SPO.IDE.A.140(d) and (e). | Aeroplanes first issued with an individual CofA on or after 1 January 2016 that have the capability to operate data link communications and are required to be equipped with a CVR shall record on a recorder, where applicable: |  |
| SPO.IDE.A.155 Flight data and cockpit voice combination recorder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - one flight data and cockpit voice combination recorder if the aeroplane has to be equipped with a CVR or an FDR - two flight data and cockpit voice combination recorders if the aeroplane has to be equipped with a CVR and an FDR. |  |  |
| SPO.IDE.A.160 Seats, seat safety belts and restraint systems ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -a seat or station for each crew member or task specialist on board; -a seat belt on each seat, and restraint devices for each station; -for other-than-complex motor-powered aeroplanes, a seat belt with upper torso restraint system on each flight crew seat, having a single point release for aeroplanes having a CofA first issued on or after 25 August 2016; -for complex motor-powered aeroplanes, a seat belt with upper torso restraint system, incorporating a device that will automatically restrain the occupant's torso in the event of rapid deceleration: - on each flight crew seat and on any seat alongside a pilot's seat; - on each observer's seat located in the flight crew compartment. - shall have a single point release; - on flight crew members seats and on any seat alongside a pilot's seat, either of the following: - two shoulder straps and a seat belt that may be used independently; - a diagonal shoulder strap and a seat belt that may be used independently for the following aeroplanes, A and B according with requirements | (A) aeroplanes with an MCTOM of 5 700 kg or less and with an MOPSC of nine or less that are compliant with the emergency landing dynamic conditions defined in the applicable certification specification; (B) aeroplanes with an MCTOM of 5 700 kg or less and with an MOPSC of nine or less that are not compliant with the emergency landing dynamic conditions defined in the applicable certification specification and having an individual CofA first issued before 25 August 2016. |  |
| SPO.IDE.A.165 First-aid kit ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| The first-aid kit shall be: - readily accessible for use - kept up-to-date. |  |  |
| SPO.IDE.A.170 Supplemental oxygen – pressurised aeroplanes ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Pressurised aeroplanes operated above flight altitudes at which the pressure altitude in the cabin compartments is above 10 000 ft shall carry enough breathing oxygen to supply all crew members and task specialists at least: - for any period when the cabin pressure altitude exceeds 15 000 ft, but in no case less than 10 minutes’ supply; - for any period when, in the event of loss of pressurisation and taking into account the circumstances of the flight, the pressure altitude in the flight crew and cabin - compartment will be between 14 000 ft and 15 000 ft; - for any period in excess of 30 minutes when the pressure altitude in the flight crew and cabin compartment will be between 10 000 ft and 14 000 ft; - for no less than 10 minutes, in the case of aeroplanes operated at pressure altitudes above 25 000 ft, or operated below that altitude, but under conditions that will not allow them to descend safely to a pressure altitude of 13 000 ft within 4 minutes. |  |  |
| Pressurised aeroplanes operated at flight altitudes above 25 000 ft shall, in addition, be equipped with: - a device to provide a warning indication to the flight crew of any loss of pressurisation; - in the case of complex motor-powered aeroplanes, quick donning masks for flight crew members. |  |  |
| SPO.IDE.A.175 Supplemental oxygen – non-pressurised aeroplanes ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Non-pressurised aeroplanes operated above flight altitudes at which the pressure altitude in the cabin compartments is above 10 000 ft shall carry enough breathing oxygen to supply: - all crew members for any period in excess of 30 minutes when the pressure altitude in the cabin compartment will be between 10 000 ft and 13 000 ft; - all persons on board for any period that the pressure altitude in the cabin compartment will be above 13 000 ft. |  |  |
| -Excursions of a specified duration between 13 000 ft and 16 000 ft may be undertaken without oxygen supplies, in accordance with SPO.OP.195(b). |  |  |
| SPO.IDE.A.180 Hand fire extinguishers ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Aeroplanes, except ELA1 aeroplanes, shall be equipped with at least one hand fire extinguisher: - in the flight crew compartment; - in each cabin compartment that is separate from the flight crew compartment, except if the compartment is readily accessible to the flight crew. -The type and quantity of extinguishing agent for the required fire extinguishers shall be suitable for the type of fire likely to occur in the compartment where the extinguisher is intended to be used and to minimise the hazard of toxic gas concentration in compartments occupied by persons. |  |  |
| SPO.IDE.A.181 Crash axe and crowbar ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Aeroplanes with an MCTOM of more than 5 700 kg shall be equipped with at least one crash axe or crowbar located in the flight crew compartment. |  |  |
| SPO.IDE.A.185 Marking of break-in points ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -If areas of the aeroplane’s fuselage suitable for break-in by rescue crews in an emergency are marked, such areas shall be marked. |  |  |
| SPO.IDE.A.190 Emergency locator transmitter ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -an ELT of any type or an aircraft localisation means meeting the requirement of Annex IV (Part CAT), CAT.GEN.MPA.210, to Regulation (EU) No 965/2012, when first issued with an individual CofA on or before 1 July 2008; -an automatic ELT or an aircraft localisation means meeting the requirement of Annex IV (Part CAT), CAT.GEN.MPA.210, to Regulation (EU) No 965/2012, when first issued with an individual CofA after 1 July 2008; - a survival ELT (ELT(S)) or a personal locator beacon (PLB), carried by a crew member or a task specialist, when certified for a maximum seating configuration of six or less. |  |  |
| - ELTs of any type and PLBs shall be capable of transmitting simultaneously on 121,5 MHz and 406 MHz. |  |  |
| SPO.IDE.A.195 Flight over water ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -single-engine landplanes when: - flying over water beyond gliding distance from land; or - taking off or landing at an aerodrome or operating site where, in the opinion of the PIC, the take-off or - approach path is so disposed over water that there would be a likelihood of a ditching; -seaplanes operated over water; and -aeroplanes operated at a distance away from land where an emergency landing is possible greater than that corresponding to 30 minutes at normal cruising speed or 50 NM, whichever is less. -Each life-jacket shall be equipped with a means of electric illumination for the purpose of facilitating the location of persons. |  |  |
| -Seaplanes operated over water shall be equipped with: - a sea anchor and other equipment necessary to facilitate mooring, anchoring or manoeuvring the aeroplane on water, appropriate to its size, weight and handling characteristics; and - equipment for making the sound signals as prescribed in the International Regulations for Preventing Collisions at Sea, where applicable. |  |  |
| -The PIC of an aeroplane operated at a distance away from land where an emergency landing is possible greater than that corresponding to 30 minutes at normal cruising speed or 50 NM, whichever is the lesser, shall determine the risks to survival of the occupants of the aeroplane in the event of a ditching, based on which he/she shall determine the carriage of: - equipment for making the distress signals; - life-rafts in sufficient numbers to carry all persons on board, stowed so as to facilitate their ready use in emergency; and - life-saving equipment, to provide the means of sustaining life, as appropriate to the flight to be undertaken. |  |  |
| SPO.IDE.A.200 Survival equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - signalling equipment to make the distress signals; - at least one survival ELT (ELT(S)); - additional survival equipment for the route to be flown taking account of the number of persons on board. |  |  |
| -The additional survival equipment specified above in does not need to be carried when the aeroplane: - remains within a distance from an area where search and rescue is not especially difficult - corresponding to: - 120 minutes at one-engine-inoperative (OEI) cruising speed for aeroplanes capable of continuing the flight to an aerodrome with the critical engine(s) becoming inoperative at any point along the route or planned diversion routes; or - 30 minutes at cruising speed for all other aeroplanes; or - remains within a distance no greater than that corresponding to 90 minutes at cruising speed from an area suitable for making an emergency landing, for aeroplanes certified in accordance with the applicable airworthiness standard. |  |  |
| SPO.IDE.A.205 Individual protective equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Each person on board shall wear individual protective equipment that is adequate for the type of operation being undertaken. |  |  |
| SPO.IDE.A.210 Headset ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Aeroplanes shall be equipped with a headset with a boom microphone or equivalent for each flight crew member at their assigned station in the flight crew compartment. - Aeroplanes operated under IFR or at night shall be equipped with a transmit button on the manual pitch and roll control for each required flight crew member.. |  |  |
| SPO.IDE.A.215 Radio communication equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Operating under IFR or at night, or when required by the applicable airspace, shall be capable of: - conducting two-way communication for aerodrome control purposes; - receiving meteorological information at any time during flight; - conducting two-way communication at any time during flight with those aeronautical stations and on those frequencies prescribed by the appropriate authority; and - providing for communication on the aeronautical emergency frequency 121,5 MHz. -When more than one communication equipment unit is required, each shall be independent of the other or others to the extent that a failure in any one will not result in failure of any other. |  |  |
| SPO.IDE.A.220 Navigation equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Aeroplanes shall be equipped with navigation equipment that will enable them to proceed in accordance with: - the ATS flight plan, if applicable; and - the applicable airspace requirements. -Aeroplanes shall have sufficient navigation equipment to ensure that, in the event of the failure of one item of equipment at any stage of the flight, the remaining equipment shall allow safe navigation in accordance, or an appropriate contingency action to be completed safely. -Aeroplanes operated on flights in which it is intended to land in IMC shall be equipped with suitable equipment capable of providing guidance to a point from which a visual landing can be performed. This equipment shall be capable of providing such guidance for each aerodrome at which it is intended to land in IMC and for any designated alternate aerodromes. -For PBN operations the aircraft shall meet the airworthiness certification requirements for the appropriate navigation specification. -Aeroplanes shall be equipped with surveillance equipment in accordance with the applicable airspace requirements. |  |  |
| SPO.IDE.A.225 Transponder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Where required by the airspace being flown, aeroplanes shall be equipped with a secondary surveillance radar (SSR) transponder with all the required capabilities. |  |  |
| SPO.IDE.A.230 Management of aeronautical databases ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Aeronautical databases used on certified aircraft system applications shall meet data quality requirements that are adequate for the intended use of the data. -The operator shall ensure the timely distribution and insertion of current and unaltered aeronautical databases to all aircraft that require them. -Notwithstanding any other occurrence reporting requirements as defined in Regulation (EU) No 376/2014, the operator shall report to the database provider instances of erroneous, inconsistent or missing data that might be reasonably expected to constitute a hazard to flight. In such cases, the operator shall inform flight crew and other personnel concerned, and shall ensure that the affected data is not used. |  |  |

| Declaração de conformidade Compliance declaration Statement Eu, _______________________________________________ declaro, sob minha responsabilidade, que o documento por mim submetido cumpre os requisitos nele inscritos e que a aeronave está conforme o aplicável no Regulamento (UE) n.º 965/2012 e que a informação prestada é verdadeira, completa e precisa. I, hereby declare, under my sole responsibility, that the document I have submitted complies with the requirements set out therein, and that the aircraft complies with the applicable requirements of Regulation (EU) No 965/2012. I further declare that the information provided is true, complete and accurate |  |  |  |
| --- | --- | --- | --- |
| Nome: Name |  | Organização: Organisation |  |
| Função: Position |  | Referência de Aprovação: Approval Reference |  |
|  |  | Assinatura e data Signature and date |  |
| Reservado aos serviços da ANAC Reserved to ANAC services |  |  |  |
| Parecer técnico ANAC : Technical Opinion – ANAC A declaração acima referida cumpre com o requerido pelo Regulamento (UE) n.º 965/2012: The aforementioned declaration complies with the requirements of Regulation (EU) No 965/2012 ☐ – Integralmente; ☐ – Com exceção dos pontos:_________________________________________________ ☐ – Não cumpre |  |  |  |
| Técnico(s) | Chefe de Departamento |  | Diretor |

ANEXO II Helicópteros

ANEXO II  Helicopters

| Requisito1 Requirement | Requisito Específico2 Specific Requirement | Método de cumprimento ou razão para não cumprimento-Operador3 Method of compliance or Reason for Non applicability-Operator |
| --- | --- | --- |
| SPO.IDE.H.105 Minimum equipment for flight ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - the helicopter is operated in accordance with the minimum equipment list (MEL); - for complex motor-powered helicopters, and for any helicopter used in commercial operations, the operator is approved by the competent authority to operate the helicopter within the constraints of the master minimum equipment list (MMEL) in accordance with point ORO.MLR.105(j) of Annex III; - the helicopter is subject to a permit to fly issued in accordance with the applicable airworthiness requirements. |  |  |
| SPO.IDE.H.115 Operating lights ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Helicopters operated at night shall be equipped with: - an anti-collision light system; - navigation/position lights; - a landing light; - lighting supplied from the helicopter’s electrical system to provide adequate illumination for all instruments and equipment essential to the safe operation of the helicopter; - lighting supplied from the helicopter’s electrical system to provide illumination in all cabin compartments; - an independent portable light for each crew member station; and - lights to conform with the International Regulations for Preventing Collisions at Sea if the helicopter is amphibious. |  |  |
| SPO.IDE.H.120 Operations under VFR – flight and navigational instruments and associated equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters operated under VFR by day shall be equipped with a means of measuring and displaying the following: - magnetic heading, - time in hours, minutes and seconds, - barometric altitude, - indicated airspeed, and - slip. |  |  |
| -Helicopters operated under VMC overwater and out of sight of the land or under VMC at night, shall be, in addition to (a), equipped with: - a means of measuring and displaying: - attitude, - vertical speed, and - stabilised heading; -a means of indicating when the supply of power to the gyroscopic instruments is not adequate; and -for complex motor-powered helicopters, a means of preventing malfunction of the airspeed indicating system required above due to condensation or icing. -a means of indicating when the supply of power to the gyroscopic instruments is not adequate; and - for complex motor-powered helicopters, a means of preventing malfunction of the airspeed indicating system required in air speed indicator due to condensation or icing. |  |  |
| - Helicopters operated when the visibility is less than 1 500 m, or in conditions where they cannot be maintained in a desired flight path without reference to one or more additional instruments, shall be, in addition to point 1 and point 2, equipped with a means of preventing malfunction of the airspeed indicating system required in airspeed indicator due to condensation or icing. |  |  |
| - Whenever two pilots are required for the operation, helicopters shall be equipped with an additional separate means of displaying: - barometric altitude, - indicated airspeed, - slip, - attitude, if applicable, - vertical speed, if applicable, and - stabilised heading, if applicable. |  |  |
| SPO.IDE.H.125 Operations under IFR – flight and navigational instruments and associated equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Helicopters under IFR shall be equipped with a means of measuring and displaying: - magnetic heading, - time in hours, minutes and seconds, - barometric altitude, - indicated airspeed, - vertical speed, - slip, - attitude, - stabilised heading, and - outside air temperature; |  |  |
| -a means of indicating when the supply of power to the gyroscopic instruments is not adequate |  |  |
| -whenever two pilots are required for the operation, an additional separate means of displaying: - barometric altitude, - indicated airspeed, - vertical speed, - slip, - attitude, and - stabilised heading; |  |  |
| d) a means of preventing malfunction of the airspeed indicating system required by indicated airspeed due to condensation or icing; |  |  |
| -an additional means of measuring and displaying attitude as a standby instrument; andthe following for complex motor-powered helicopters: - an alternate source of static pressure; and - a chart holder in an easily readable position that can be illuminated for night operations |  |  |
| SPO.IDE.H.126 - Additional equipment for single-pilot operation under IFR ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters operated under IFR with a single pilot shall be equipped with an autopilot with at least altitude hold and heading mode. |  |  |
| SPO.IDE.H.132 Airborne weather detecting equipment – complex motor-powered helicopters ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Helicopters operated under IFR or at night shall be equipped with airborne weather detecting equipment when current weather reports indicate that thunderstorms or other potentially hazardous weather conditions, regarded as detectable with airborne weather detecting equipment, may be expected to exist along the route to be flown. |  |  |
| SPO.IDE.H.133 Additional equipment for operations in icing conditions at night – complex motor-powered helicopters ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Helicopters operated in expected or actual icing conditions at night shall be equipped with a means to illuminate or detect the formation of ice. - The means to illuminate the formation of ice shall not cause glare or reflection that would handicap flight crew members in the performance of their duties. |  |  |
| SPO.IDE.H.135 Flight crew interphone system ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Helicopters operated by more than one flight crew member shall be equipped with a flight crew interphone system, including headsets and microphones for use by all flight crew members.. |  |  |
| SPO.IDE.H.140 Cockpit voice recorder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters with an MCTOM of more than 7 000 kg and first issued with an individual CofA on or after 1 January 2016 shall be equipped with a CVR. -The CVR shall be capable of retaining data recorded during at least the preceding 2 hours. -The CVR shall record with reference to a timescale: - voice communications transmitted from or received in the flight crew compartment by radio; - flight crew members’ voice communications using the interphone system and the public address system, if installed; - the aural environment of the cockpit, including, without interruption, the audio signals received from each crew microphone; and - voice or audio signals identifying navigation or approach aids introduced into a headset or speaker. |  |  |
| The CVR shall start automatically to record prior to the helicopter moving under its own power and shall continue to record until the termination of the flight when the helicopter is no longer capable of moving under its own power. |  |  |
| In addition to (d), depending on the availability of electrical power, the CVR shall start to record as early as possible during the cockpit checks prior to engine start at the beginning of the flight until the cockpit checks immediately following engine shutdown at the end of the flight. |  |  |
| If the CVR is not deployable, it shall have a device to assist in locating it under water. By 1 January 2020 at the latest, this device shall have a minimum underwater transmission time of 90 days. If the CVR is deployable, it shall have an automatic emergency locator transmitter. |  |  |
| SPO.IDE.H.145 Flight data recorder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters with an MCTOM of more than 3 175 kg and first issued with an individual CofA on or after 1 January 2016 shall be equipped with an FDR that uses a digital method of recording and storing data and for which a method of readily retrieving that data from the storage medium is available. - The FDR shall record the parameters required to determine accurately the helicopter flight path, speed, attitude, engine power, configuration and operation and be capable of retaining data recorded during at least the preceding 10 hours. - Data shall be obtained from helicopter sources that enable accurate correlation with information displayed to the flight crew. - The FDR shall start automatically to record the data prior to the helicopter being capable of moving under its own power and shall stop automatically after the helicopter is incapable of moving under its own power. - If the FDR is not deployable, it shall have a device to assist in locating it under water. By 1 January 2020 at the latest, this device shall have a minimum underwater transmission time of 90 days. If the FDR is deployable, it shall have an automatic emergency locator transmitter. |  |  |
| SPO.IDE.H.146 Lightweight flight recorder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Turbine-engined helicopters with an MCTOM of 2 250 kg or more shall be equipped with a flight recorder if all the following conditions are met: - they are not within the scope of point SPO.IDE.H.145(a); - they are used for commercial operations; - they are first issued with an individual CofA on or after 5 September 2022. |  |  |
| -The flight recorder shall record, by means of flight data or images, information that is sufficient to determine the flight path and aircraft speed. -The flight recorder shall be capable of retaining the flight data and the images recorded during at least the preceding 5 hours. - The flight recorder shall automatically start to record prior to the helicopter being capable of moving under its own power and shall stop automatically after the helicopter is no longer capable of moving under its own power. - If the flight recorder records images or audio of the flight crew compartment, then a function shall be provided which can be operated by the pilot-in-command and which modifies image and audio recordings made before the operation of that function, so that those recordings cannot be retrieved using normal replay or copying techniques. |  |  |
| SPO.IDE.H.150 Data link recording ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -data link communication messages related to ATS communications to and from the helicopter, including messages applying to the following applications: - data link initiation; - controller-pilot communication; - addressed surveillance; - flight information; - as far as is practicable, given the architecture of the system, aircraft broadcast surveillance; - as far as is practicable, given the architecture of the system, aircraft operational control data; and - as far as is practicable, given the architecture of the system, graphics; -information that enables correlation to any associated records related to data link communications and stored separately from the helicopter; and -information on the time and priority of data link communications messages, taking into account the system’s architecture. | Helicopters first issued with an individual CofA on or after 1 January 2016 that have the capability to operate data link communications and are required to be equipped with a CVR shall record on a recorder, where applicable: |  |
| -The recorder shall use a digital method of recording and storing data and information and a method for readily retrieving that data. The recording method shall allow the data to match the data recorded on the ground. |  |  |
| -The recorder shall be capable of retaining data recorded for at least the same duration as set out for CVRs in SPO.IDE.H.140. |  |  |
| If the recorder is not deployable, it shall have a device to assist in locating it under water. By 1 January 2020 at the latest, this device shall have a minimum underwater transmission time of 90 days. If the recorder is deployable, it shall have an automatic emergency locator transmitter. |  |  |
| -The requirements applicable to the start and stop logic of the recorder are the same as the requirements applicable to the start and stop logic of the CVR contained in SPO.IDE.H.140 (d) and (e). |  |  |
| SPO.IDE.H.155 Flight data and cockpit voice combination recorder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Compliance with CVR and FDR requirements may be achieved by one flight data and cockpit voice combination recorder. |  |  |
| SPO.IDE.H.160 Seats, seat safety belts and restraint systems ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters shall be equipped with: - a seat or station for each crew member or task specialist on board; - a seat belt on each seat, and restraint devices for each station; - for helicopters first issued with an individual CofA after 31 December 2012, a seat belt with an upper torso restraint system for each seat; and, - a seat belt with upper torso restraint system incorporating a device that will automatically restrain the occupant’s torso in the event of rapid deceleration on each flight crew seat -A seat belt with upper torso restraint system shall have a single point release. |  |  |
| SPO.IDE.H.165 First-aid kit ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| The first-aid kit shall be: - readily accessible for use - kept up-to-date. |  |  |
| SPO.IDE.H.175 Supplemental oxygen – non-pressurised helicopters ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Non-pressurised helicopters operated at flight altitudes when the oxygen supply is required in accordance with (b) shall be equipped with oxygen storage and dispensing apparatus capableof storing and dispensing the required oxygen supplies. -Non-pressurised helicopters operated above flight altitudes at which the pressure altitude in the cabin compartments is above 10 000 ft shall carry enough breathing oxygen to supply: - all crew members for any period in excess of 30 minutes when the pressure altitude inthe cabin compartment will be between 10 000 ft and 13 000 ft; and - all crew members and task specialists for any period that the pressure altitude in the cabin compartment will be above 13 000 ft. |  |  |
| -Excursions of a specified duration between 13 000 ft and 16 000 ft may be undertaken without oxygen supplies, in accordance with SPO.OP.195(b). |  |  |
| SPO.IDE.H.180 Hand fire extinguishers ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Helicopters, except ELA2 helicopters, shall be equipped with at least one hand fire extinguisher: - in the flight crew compartment; and - in each cabin compartment that is separate from the flight crew compartment, except if the compartment is readily accessible to the flight crew. -The type and quantity of extinguishing agent for the required fire extinguishers shall be suitable for the type of fire likely to occur in the compartment where the extinguisher is intended to be used and to minimise the hazard of toxic gas concentration in compartments occupied by persons. |  |  |
| SPO.IDE.H.185 Marking of break-in points ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -If areas of the helicopter’s fuselage suitable for break-in by rescue crews in an emergency are marked, such areas shall be marked. |  |  |
| SPO.IDE.H.190 Emergency locator transmitter ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters certified for a maximum seating configuration above six shall be equipped with: - an automatic ELT; and - one survival ELT (ELT(S)) in a life-raft or life-jacket when the helicopter is operated at a distance from land corresponding to more than 3 minutes flying time at normal cruising speed. |  |  |
| -Helicopters certified for a maximum seating configuration of six or less shall be equipped with an ELT(S) or a personal locator beacon (PLB), carried by a crew member or a task specialist, or with an automatic ELT. |  |  |
| - ELTs of any type and PLBs shall be capable of transmitting simultaneously on 121,5 MHz and 406 MHz. |  |  |
| SPO.IDE.H.195 Flight over water – other-than complex motor-powered Helicopters ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters shall be equipped with a life-jacket for each person on board, that shall be worn or stowed in a position that is readily accessible from the seat or station of the person for whose use it is provided, when: - flying over water beyond auto rotational distance from the land where in case of the critical engine failure, the helicopter is not able to sustain level flight; or - flying over water at a distance of land corresponding to more than 10 minutes flying at normal cruising speed, where in case of the critical engine failure, the helicopter is able to sustain level flight; or - taking off or landing at an aerodrome/operating site where the take-off or approach path is over water. |  |  |
| -Each life-jacket shall be equipped with a means of electric illumination for the purpose of facilitating the location of persons. |  |  |
| -The pilot-in-command of a helicopter operated on a flight over water at a distance from land corresponding to more than 30 minutes flying time at normal cruising speed or 50 NM, whichever is less, shall determine the risks to survival of the occupants of the helicopter in the event of a ditching, based on which he/she shall determine the carriage of: - equipment for making the distress signals; - life-rafts in sufficient numbers to carry all persons on board, stowed so as to facilitate their ready use in emergency; and - life-saving equipment to provide the means of sustaining life, as appropriate to the flight to be undertaken. |  |  |
| -The pilot-in-command shall determine the risks to survival of the occupants of the helicopter in the event of a ditching, when deciding if the life-jackets required in (a) shall be worn by all occupants. |  |  |
| SPO.IDE.H.197 Life-jackets – complex motor-powered helicopters ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters shall be equipped with a life-jacket for each person on board, that shall be worn or stowed in a position that is readily accessible from the seat or station of the person for whose use it is provided, when: - operated on a flight over water at a distance from land corresponding to more than 10 minutes flying time at normal cruising speed, where in the case of the critical engine failure, the helicopter is able to sustain level flight; - operated on a flight over water beyond auto-rotational distance from the land, where in the case of the critical engine failure, the helicopter is not able to sustain level flight; or - taking off or landing at an aerodrome or operating site where the take-off or approach path is so disposed over water that in the event of a mishap there would be the likelihood of a ditching. |  |  |
| -Each life-jacket shall be equipped with a means of electric illumination for the purpose of facilitating the location of persons. |  |  |
| SPO.IDE.H.198 Survival suits – complex motor-powered helicopters ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Each person on board shall wear a survival suit when so determined by the pilot-in-command based on a risk assessment taking into account the following conditions: - flights over water beyond autorotational distance or safe forced-landing distance from land, where, in the case of a critical engine failure, the helicopter is not able to sustain level flight; and - the weather report or forecasts available to the pilot-in-command indicate that the sea temperature will be less than plus 10 °C during the flight. |  |  |
| SPO.IDE.H.199 Life-rafts, survival ELTs and survival equipment on extended overwater flights – complex motor-powered helicopters Reg. (EU) No 379/201 4 ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -on a flight over water at a distance from land corresponding to more than 10 minutes flying time at normal cruising speed where in the case of the critical engine failure, the helicopter is able to sustain level flight; or - on a flight over water at a distance corresponding to more than 3 minutes flying time at normal cruising speed, where in the case of the critical engine failure, the helicopter is not able to sustain level flight, and if so determined by the pilot-in-command by means of a risk assessment, shall be equipped with: - at least one life-raft with a rated capacity of not less than the maximum number of persons on board, stowed so as to facilitate their ready use in emergency; - at least one survival ELT (ELT(S)) for each required life-raft; and - life-saving equipment, including means of sustaining life, as appropriate to the flight to be undertaken. |  |  |
| SPO.IDE.H.200 Survival equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters operated over areas in which search and rescue would be especially difficult shall be equipped with: - signalling equipment to make distress signals; - at least one survival ELT (ELT(S)); and - additional survival equipment for the route to be flown taking account of the number of persons on board. |  |  |
| SPO.IDE.H.202 Helicopters certified for operating on water – miscellaneous equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| - Helicopters certified for operating on water shall be equipped with: - a sea anchor and other equipment necessary to facilitate mooring, anchoring or manoeuvring the helicopter on water, appropriate to its size, weight and handling characteristics; and - equipment for making the sound signals prescribed in the International Regulations for Preventing Collisions at Sea, where applicable. |  |  |
| SPO.IDE.H.203 All helicopters on flights over water – ditching ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Complex motor-powered helicopters operated on a flight over water in a hostile environment at a distance from land corresponding to more than 10 minutes' flying time at normal cruising speed and other-than complex motor-powered helicopters flying over water in a hostile environment beyond a distance of 50 NM from land shall be: - designed for landing on water in accordance with the relevant airworthiness code; - certified for ditching in accordance with the relevant airworthiness code; or - fitted with emergency flotation equipment |  |  |
| SPO.IDE.H.205 Individual protective equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Each person on board shall wear individual protective equipment that is adequate for the type of operation being undertaken. |  |  |
| SPO.IDE.H.210 Headset ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Whenever a radio communication and/or radio navigation system is required, helicopters shall be equipped with a headset with boom microphone or equivalent and a transmit button on the flight controls for each required pilot, crew member and/or task specialist at his/her assigned station. |  |  |
| SPO.IDE.H.215 Radio communication equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters operated under IFR or at night, or when required by the applicable airspace requirements, shall be equipped with radio communication equipment that, under normal radio propagating conditions, shall be capable of: - conducting two-way communication for aerodrome control purposes; - receiving meteorological information; - conducting two-way communication at any time during flight with those aeronauticalstations and on those frequencies prescribed by the appropriate authority; and - providing for communication on the aeronautical emergency frequency 121,5 MHz. -When more than one communications equipment unit is required, each shall be independent of the other or others to the extent that a failure in any one will not result in failure of any other. - When a radio communication system is required, and in addition to the flight crew interphone system required in SPO.IDE.H.135, helicopters shall be equipped with a transmit button on the flight controls for each required pilot and crew member at his/her assigned station. |  |  |
| SPO.IDE.H.220 Navigation equipment ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Helicopters shall be equipped with navigation equipment that will enable them to proceed in accordance with: - the ATS flight plan, if applicable; and - the applicable airspace requirements. -Helicopters shall have sufficient navigation equipment to ensure that, in the event of the failure of one item of equipment at any stage of the flight, the remaining equipment shall allow safe navigation in accordance with (a), or an appropriate contingency action to be completed safely. - Helicopters operated on flights in which it is intended to land in IMC shall be equipped with navigation equipment capable of providing guidance to a point from which a visual landing can be performed. This equipment shall be capable of providing such guidance for each aerodrome at which it is intended to land in IMC and for any designated alternate aerodromes. - For PBN operations the aircraft shall meet the airworthiness certification requirements for the appropriate navigation specification. - Helicopters shall be equipped with surveillance equipment in accordance with the applicable airspace requirements. |  |  |
| SPO.IDE.H.225 Transponder ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| Where required by the airspace being flown, helicopters shall be equipped with a secondary surveillance radar (SSR) transponder with all the required capabilities. |  |  |
| SPO.IDE.H.230 Management of aeronautical databases ☐ N/A ☐ Compliant ☐ Non Compliant |  |  |
| -Aeronautical databases used on certified aircraft system applications shall meet data quality requirements that are adequate for the intended use of the data. -The operator shall ensure the timely distribution and insertion of current and unaltered aeronautical databases to all aircraft that require them. - Notwithstanding any other occurrence reporting requirements as defined in Regulation (EU) No 376/2014, the operator shall report to the database provider instances of erroneous, inconsistent or missing data that might be reasonably expected to constitute a hazard to flight. |  |  |

| Declaração de conformidade Compliance declaration Statement Eu, _______________________________________________ declaro, sob minha responsabilidade, que o documento por mim submetido cumpre os requisitos nele inscritos e que a aeronave está conforme o aplicável no Regulamento (UE) n.º 965/2012 e que a informação prestada é verdadeira, completa e precisa. I, hereby declare, under my sole responsibility, that the document I have submitted complies with the requirements set out therein, and that the aircraft complies with the applicable requirements of Regulation (EU) No 965/2012 I further declare that the information provided is true, complete and accurate |  |  |  |
| --- | --- | --- | --- |
| Nome: Name |  | Organização: Organisation |  |
| Função: Position |  | Referência de Aprovação: Approval Reference |  |
|  |  | Assinatura e data Signature and date |  |
| Reservado aos serviços da ANAC Reserved to ANAC services |  |  |  |
| Parecer técnico ANAC : Technical Opinion – ANAC A declaração acima referida cumpre com o requerido pelo Regulamento (UE) n.º 965/2012: The aforementioned declaration complies with the requirements of Regulation (EU) No 965/2012 ☐ – Integralmente; ☐ – Com exeção dos pontos:_________________________________________________ ☐ – Não cumpre |  |  |  |
| Técnico(s) | Chefe de Departamento |  | Diretor |